TRAVISTILLEY.COM / TRAINING CENTER Electrical Safety for Non-Qualified Employees INSTRUCTOR + PRINT PACKET

| AUDIENCE | EST. TIME | REFERENCE |
| --- | --- | --- |
| Employees who may face electrical hazards | 30-40 min | 29 CFR 1910.332 / 1910 Subpart S |

IMPORTANT USE NOTE Awareness for non-qualified employees. Only qualified persons may perform work that requires the knowledge/skills and permissions assigned to qualified electrical workers.

# What is inside this packet

Instructor quick-start and site-specific preparation checklist

Slide-by-slide trainer guide with discussion prompts

Employee handout / key takeaways

Printable 10-question knowledge check

Separate answer key and coaching notes

Attendance record

Practical / workplace evaluation checklist

Certificate wording and completion record

Primary regulatory/source reference

INSTRUCTOR QUICK-START

# Run a strong session without overcomplicating it.

| 1. PREPARE | Review the deck and packet. Insert your procedures, equipment, routes, photos, emergency contacts and site rules. |
| --- | --- |
| 2. PRESENT | Use the PowerPoint/PDF on a screen. Pause for discussion rather than reading every bullet. |
| 3. CHECK | Give the knowledge check. Review missed items and coach to the expected decision. |
| 4. VERIFY | Complete required hands-on or workplace-specific practice/evaluation with an appropriate trainer/evaluator. |
| 5. DOCUMENT | Use the attendance record, evaluation and certificate only after required training elements are complete. |

BEFORE EMPLOYEES WORK Awareness for non-qualified employees. Only qualified persons may perform work that requires the knowledge/skills and permissions assigned to qualified electrical workers.

## Learning objectives

Recognize common electrical hazards and damaged equipment

Distinguish non-qualified versus qualified work boundaries

Use cords, plugs and portable equipment safely

Respond safely to shock, fire, wet conditions and exposed energized parts

TRAINER GUIDE / MODULES 1-2

# 1. Know your role

Non-qualified employees need safety practices necessary for their work but are not authorized to perform qualified electrical tasks.

Do not open panels, defeat interlocks or work on exposed energized parts unless specifically qualified/authorized.

DISCUSSION PROMPT Where could this fail here?

# 2. Cords, plugs and tools

Inspect for cut insulation, missing ground pins, damaged strain relief, exposed conductors and heat damage.

Remove defective equipment from service.

Use equipment rated for the environment and follow GFCI/assured grounding requirements where applicable.

DISCUSSION PROMPT What control do we use?

TRAINER GUIDE / MODULES 3-4

# 3. Panels and access

Keep electrical equipment access clear.

Do not store materials in required working space.

Report open covers, missing blanks, damaged enclosures or signs of overheating.

DISCUSSION PROMPT What would make you stop?

# 4. Wet locations and grounding

Water increases shock risk.

Use approved equipment/protection for wet/damp locations.

Never improvise adapters or defeat grounding.

DISCUSSION PROMPT Where could this fail here?

TRAINER GUIDE / MODULES 5-6

# 5. Overhead and exposed parts

Maintain required clearance from overhead/exposed energized parts based on employer procedure and applicable standard.

Use barriers/qualified assistance when work could encroach.

DISCUSSION PROMPT What control do we use?

# 6. Emergency response

Do not touch a person who is still in contact with electricity until the source is safely controlled.

Call emergency response and use trained rescue/first-aid procedures.

Electrical fires may require specific extinguisher selection and de-energization.

DISCUSSION PROMPT What would make you stop?

FACILITATED DISCUSSION

# Stop-work / red-flag conditions

Damaged cord/plug or missing ground pin

Open electrical panel or exposed live parts

Wet work near non-rated electrical equipment

Employee preparing to reset/work inside equipment beyond their qualification

SCENARIO A worker finds an extension cord with a cut jacket and visible conductor but says it only powers a small fan. What should happen?

RECOMMENDED RESPONSE Remove the cord from service immediately and replace/repair it through an approved process; load size does not make exposed conductors safe.

## Make it site-specific

Define what non-qualified employees may and may not do

Show defect tagging/removal process

Review GFCI/wet-location rules and extension-cord policy

Identify restricted electrical areas and emergency response

EMPLOYEE HANDOUT

# Electrical Safety for Non-Qualified Employees

KEEP THIS PAGE

## Key decisions to remember

Know your role: Non-qualified employees need safety practices necessary for their work but are not authorized to perform qualified electrical tasks.

Cords, plugs and tools: Inspect for cut insulation, missing ground pins, damaged strain relief, exposed conductors and heat damage.

Panels and access: Keep electrical equipment access clear.

Wet locations and grounding: Water increases shock risk.

Overhead and exposed parts: Maintain required clearance from overhead/exposed energized parts based on employer procedure and applicable standard.

Emergency response: Do not touch a person who is still in contact with electricity until the source is safely controlled.

## Stop and get help when...

Damaged cord/plug or missing ground pin

Open electrical panel or exposed live parts

Wet work near non-rated electrical equipment

Employee preparing to reset/work inside equipment beyond their qualification

## Our site-specific information

☐ Define what non-qualified employees may and may not do

☐ Show defect tagging/removal process

☐ Review GFCI/wet-location rules and extension-cord policy

☐ Identify restricted electrical areas and emergency response

Employee notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

EMPLOYEE KNOWLEDGE CHECK

# Electrical Safety for Non-Qualified Employees - Knowledge Check

Employee: ____________________________________________      Date: ____________________

1. A non-qualified employee should:

☐ A. Perform only electrical-related work they are trained/authorized to do

☐ B. Open energized panels when careful

☐ C. Repair cords with tape whenever needed

☐ D. Bypass interlocks

2. A cord with exposed conductor should be:

☐ A. Removed from service

☐ B. Used for low-power tools only

☐ C. Covered with paper

☐ D. Used indoors only

3. Electrical panel working space should be:

☐ A. Kept clear

☐ B. Used for storage

☐ C. Blocked during production

☐ D. Covered with pallets

4. A missing grounding pin is:

☐ A. A defect that requires the equipment/cord to be removed from use

☐ B. Acceptable on dry days

☐ C. Safe with rubber shoes

☐ D. Only cosmetic

5. Wet conditions generally:

☐ A. Increase electrical shock risk

☐ B. Eliminate static electricity only

☐ C. Make grounding unnecessary

☐ D. Reduce voltage

Score: ______ / 10     Passing requirement / retraining criteria: ______________________________________________

EMPLOYEE KNOWLEDGE CHECK / CONTINUED

6. If a person is in contact with energized equipment, a rescuer should:

☐ A. Avoid touching them until the electrical source is safely controlled

☐ B. Grab them immediately

☐ C. Throw water on the equipment

☐ D. Use a metal tool

7. Qualified electrical work requires:

☐ A. Specific knowledge, training and employer authorization

☐ B. Only seniority

☐ C. Any safety orientation

☐ D. A written quiz only

8. Overhead electrical hazards require:

☐ A. Maintaining safe clearance and following applicable procedures

☐ B. Relying on equipment tires

☐ C. Touching wires with a stick

☐ D. Ignoring lines below 600V

9. Defective electrical equipment should be:

☐ A. Tagged/removed from service per employer procedure

☐ B. Used by supervisors only

☐ C. Used if inspected verbally

☐ D. Stored energized

10. This awareness course makes someone a qualified electrical person.

☐ A. False

☐ B. True

Employee signature: _____________________________________________

INSTRUCTOR ONLY

# Answer key + coaching notes

1. A - Perform only electrical-related work they are trained/authorized to do  |  A non-qualified employee should:

2. A - Removed from service  |  A cord with exposed conductor should be:

3. A - Kept clear  |  Electrical panel working space should be:

4. A - A defect that requires the equipment/cord to be removed from use  |  A missing grounding pin is:

5. A - Increase electrical shock risk  |  Wet conditions generally:

6. A - Avoid touching them until the electrical source is safely controlled  |  If a person is in contact with energized equipment, a rescuer should:

7. A - Specific knowledge, training and employer authorization  |  Qualified electrical work requires:

8. A - Maintaining safe clearance and following applicable procedures  |  Overhead electrical hazards require:

9. A - Tagged/removed from service per employer procedure  |  Defective electrical equipment should be:

10. A - False  |  This awareness course makes someone a qualified electrical person.

COACHING RULE Do not treat a passing written score as proof of hands-on competency. Retrain missed critical concepts and complete workplace-specific verification where required.

TRAINING RECORD

# Attendance + completion record

Course: Electrical Safety for Non-Qualified Employees

Date: ____________   Instructor: ______________________________   Department / Location: ______________________________

| Employee Printed Name | Employee Signature | Job / Department | Date |
| --- | --- | --- | --- |

Site-specific topics / notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

PRACTICAL / WORKPLACE VERIFICATION

# Can the employee apply it?

Employee: ______________________________   Evaluator: ______________________________   Date: ____________

| Evaluation Item | Meets | Coach | N/A |
| --- | --- | --- | --- |
| Inspects cord/tool | ☐ | ☐ | ☐ |
| Identifies panel clearance problem | ☐ | ☐ | ☐ |
| Explains non-qualified boundary | ☐ | ☐ | ☐ |
| Reports damaged equipment | ☐ | ☐ | ☐ |
| Uses GFCI/site protection correctly | ☐ | ☐ | ☐ |
| Describes electrical emergency action | ☐ | ☐ | ☐ |

Comments / coaching:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Result:  ☐ Competent for evaluated task (subject to employer authorization)   ☐ More training/practice needed

Evaluator signature: ______________________________   Employee signature: ______________________________

COMPLETION CERTIFICATE

CERTIFICATE OF TRAINING

This certifies that
____________________________________________

completed the formal / awareness training portion of
Electrical Safety for Non-Qualified Employees

Date: ____________________     Instructor: ____________________
Employer / Location: __________________________________________

Awareness for non-qualified employees. Only qualified persons may perform work that requires the knowledge/skills and permissions assigned to qualified electrical workers.

REFERENCE / FINAL TRAINER CHECK

# Regulatory / source reference

Primary reference: 29 CFR 1910.332 / 1910 Subpart S

https://www.osha.gov/laws-regs/regulations/standardnumber/1910/1910.332

## Before delivery

Confirm current OSHA and applicable state-plan requirements

Confirm manufacturer instructions and employer procedures

Insert workplace-specific hazards, controls, emergency contacts and rules

Use competent/qualified trainers or evaluators where required

Complete practical/workplace training and authorization requirements where applicable

Retain training records according to employer and regulatory requirements

FINAL REMINDER Awareness for non-qualified employees. Only qualified persons may perform work that requires the knowledge/skills and permissions assigned to qualified electrical workers.