TRAVISTILLEY.COM / TRAINING CENTER Fall Protection Fundamentals INSTRUCTOR + PRINT PACKET

| AUDIENCE | EST. TIME | REFERENCE |
| --- | --- | --- |
| Employees exposed to fall hazards | 35-45 min | 29 CFR 1926.503 / applicable walking-working surface rules |

IMPORTANT USE NOTE General fall-hazard instruction. Employer training must address the fall hazards and protection systems used in the actual work area and applicable industry standard.

# What is inside this packet

Instructor quick-start and site-specific preparation checklist

Slide-by-slide trainer guide with discussion prompts

Employee handout / key takeaways

Printable 10-question knowledge check

Separate answer key and coaching notes

Attendance record

Practical / workplace evaluation checklist

Certificate wording and completion record

Primary regulatory/source reference

INSTRUCTOR QUICK-START

# Run a strong session without overcomplicating it.

| 1. PREPARE | Review the deck and packet. Insert your procedures, equipment, routes, photos, emergency contacts and site rules. |
| --- | --- |
| 2. PRESENT | Use the PowerPoint/PDF on a screen. Pause for discussion rather than reading every bullet. |
| 3. CHECK | Give the knowledge check. Review missed items and coach to the expected decision. |
| 4. VERIFY | Complete required hands-on or workplace-specific practice/evaluation with an appropriate trainer/evaluator. |
| 5. DOCUMENT | Use the attendance record, evaluation and certificate only after required training elements are complete. |

BEFORE EMPLOYEES WORK General fall-hazard instruction. Employer training must address the fall hazards and protection systems used in the actual work area and applicable industry standard.

## Learning objectives

Recognize common fall exposures and work-at-height hazards

Differentiate guardrails, travel restraint, personal fall arrest and other systems

Inspect and use fall-protection equipment correctly

Understand anchorage, clearance, swing-fall and rescue considerations

TRAINER GUIDE / MODULES 1-2

# 1. Recognize the exposure

Open sides/edges, floor holes, roofs, ladders, scaffolds, lifts and elevated platforms can create fall hazards.

The required protection depends on work type, height, industry and specific standard.

DISCUSSION PROMPT Where could this fail here?

# 2. Prevent before arrest

Guardrails, covers and other passive protection often prevent exposure.

Travel restraint keeps a worker from reaching the edge.

Personal fall arrest is designed to stop a fall after it begins and requires a complete system.

DISCUSSION PROMPT What control do we use?

TRAINER GUIDE / MODULES 3-4

# 3. Personal fall arrest system

Use compatible full-body harness, connector and approved anchorage/system.

Consider total fall clearance, free-fall distance and swing-fall exposure.

Never anchor to an unapproved point.

DISCUSSION PROMPT What would make you stop?

# 4. Inspection and fit

Inspect harness/webbing, D-rings, stitching, snap hooks, lanyards/SRLs and labels before use.

Adjust harness for correct fit and remove damaged/impact-loaded equipment from service per program/manufacturer.

DISCUSSION PROMPT Where could this fail here?

TRAINER GUIDE / MODULES 5-6

# 5. Work practices

Maintain three points of contact where required, keep walking surfaces clear and protect openings.

Do not climb on rails or improvise platforms.

Control tools/materials that could fall on people below.

DISCUSSION PROMPT What control do we use?

# 6. Rescue and training

Plan for prompt rescue before using personal fall arrest.

Workers need training in the hazards and systems they will use.

Retraining is needed when changes or performance show the worker lacks understanding/skill.

DISCUSSION PROMPT What would make you stop?

FACILITATED DISCUSSION

# Stop-work / red-flag conditions

No clear approved anchorage

Worker cannot calculate/confirm fall clearance

Damaged or unlabeled fall-protection equipment

No rescue plan for a worker who could remain suspended

SCENARIO A worker ties a lanyard to a nearby pipe because it is “strong enough.” What is wrong?

RECOMMENDED RESPONSE Fall-protection anchorage must be approved/designed for the system and task; workers cannot improvise anchor points based only on appearance.

## Make it site-specific

Identify actual fall hazards and applicable threshold/standard

Show approved systems and anchorages

Demonstrate harness fit, connection and equipment inspection

Review rescue procedure and dropped-object controls

EMPLOYEE HANDOUT

# Fall Protection Fundamentals

KEEP THIS PAGE

## Key decisions to remember

Recognize the exposure: Open sides/edges, floor holes, roofs, ladders, scaffolds, lifts and elevated platforms can create fall hazards.

Prevent before arrest: Guardrails, covers and other passive protection often prevent exposure.

Personal fall arrest system: Use compatible full-body harness, connector and approved anchorage/system.

Inspection and fit: Inspect harness/webbing, D-rings, stitching, snap hooks, lanyards/SRLs and labels before use.

Work practices: Maintain three points of contact where required, keep walking surfaces clear and protect openings.

Rescue and training: Plan for prompt rescue before using personal fall arrest.

## Stop and get help when...

No clear approved anchorage

Worker cannot calculate/confirm fall clearance

Damaged or unlabeled fall-protection equipment

No rescue plan for a worker who could remain suspended

## Our site-specific information

☐ Identify actual fall hazards and applicable threshold/standard

☐ Show approved systems and anchorages

☐ Demonstrate harness fit, connection and equipment inspection

☐ Review rescue procedure and dropped-object controls

Employee notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

EMPLOYEE KNOWLEDGE CHECK

# Fall Protection Fundamentals - Knowledge Check

Employee: ____________________________________________      Date: ____________________

1. Fall protection should be selected based on:

☐ A. The actual hazard, task and applicable standard

☐ B. Employee preference only

☐ C. The lowest-cost device

☐ D. A universal height for all work

2. Travel restraint is intended to:

☐ A. Keep a worker from reaching the fall edge

☐ B. Arrest a fall after it occurs

☐ C. Replace guardrails automatically

☐ D. Serve as a rescue system

3. Personal fall arrest requires attention to:

☐ A. Anchorage, connectors, harness, clearance and rescue

☐ B. Only harness color

☐ C. Only lanyard length

☐ D. Only shoe type

4. A worker should anchor to:

☐ A. An approved anchorage/system for the task

☐ B. Any pipe that looks strong

☐ C. A handrail without approval

☐ D. Another worker

5. Fall-protection equipment should be inspected:

☐ A. Before use and as required by the employer/manufacturer

☐ B. Only after a fall

☐ C. Only annually

☐ D. Only by the vendor

Score: ______ / 10     Passing requirement / retraining criteria: ______________________________________________

EMPLOYEE KNOWLEDGE CHECK / CONTINUED

6. Damaged or impact-loaded equipment should be:

☐ A. Removed from service and handled per program/manufacturer

☐ B. Used for light duty

☐ C. Shared with another worker

☐ D. Hidden in storage

7. A rescue plan matters because:

☐ A. Suspended workers may need prompt rescue after a fall

☐ B. It increases tool storage

☐ C. It replaces training

☐ D. It eliminates clearance needs

8. Standing on guardrails is:

☐ A. Unsafe and not an acceptable way to gain height

☐ B. Allowed below 10 feet

☐ C. Allowed with safety glasses

☐ D. Required for roof work

9. Employees exposed to fall hazards should be trained to:

☐ A. Recognize hazards and use the required procedures/systems

☐ B. Only sign attendance

☐ C. Choose their own anchorages

☐ D. Skip inspection after experience

10. This course replaces site-specific fall-protection instruction.

☐ A. False

☐ B. True

Employee signature: _____________________________________________

INSTRUCTOR ONLY

# Answer key + coaching notes

1. A - The actual hazard, task and applicable standard  |  Fall protection should be selected based on:

2. A - Keep a worker from reaching the fall edge  |  Travel restraint is intended to:

3. A - Anchorage, connectors, harness, clearance and rescue  |  Personal fall arrest requires attention to:

4. A - An approved anchorage/system for the task  |  A worker should anchor to:

5. A - Before use and as required by the employer/manufacturer  |  Fall-protection equipment should be inspected:

6. A - Removed from service and handled per program/manufacturer  |  Damaged or impact-loaded equipment should be:

7. A - Suspended workers may need prompt rescue after a fall  |  A rescue plan matters because:

8. A - Unsafe and not an acceptable way to gain height  |  Standing on guardrails is:

9. A - Recognize hazards and use the required procedures/systems  |  Employees exposed to fall hazards should be trained to:

10. A - False  |  This course replaces site-specific fall-protection instruction.

COACHING RULE Do not treat a passing written score as proof of hands-on competency. Retrain missed critical concepts and complete workplace-specific verification where required.

TRAINING RECORD

# Attendance + completion record

Course: Fall Protection Fundamentals

Date: ____________   Instructor: ______________________________   Department / Location: ______________________________

| Employee Printed Name | Employee Signature | Job / Department | Date |
| --- | --- | --- | --- |

Site-specific topics / notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

PRACTICAL / WORKPLACE VERIFICATION

# Can the employee apply it?

Employee: ______________________________   Evaluator: ______________________________   Date: ____________

| Evaluation Item | Meets | Coach | N/A |
| --- | --- | --- | --- |
| Identifies fall hazards | ☐ | ☐ | ☐ |
| Selects correct protection method | ☐ | ☐ | ☐ |
| Inspects harness/connectors | ☐ | ☐ | ☐ |
| Adjusts harness correctly | ☐ | ☐ | ☐ |
| Connects to approved system | ☐ | ☐ | ☐ |
| Explains clearance/swing fall | ☐ | ☐ | ☐ |
| Maintains safe work position | ☐ | ☐ | ☐ |
| Explains rescue/reporting steps | ☐ | ☐ | ☐ |

Comments / coaching:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Result:  ☐ Competent for evaluated task (subject to employer authorization)   ☐ More training/practice needed

Evaluator signature: ______________________________   Employee signature: ______________________________

COMPLETION CERTIFICATE

CERTIFICATE OF TRAINING

This certifies that
____________________________________________

completed the formal / awareness training portion of
Fall Protection Fundamentals

Date: ____________________     Instructor: ____________________
Employer / Location: __________________________________________

General fall-hazard instruction. Employer training must address the fall hazards and protection systems used in the actual work area and applicable industry standard.

REFERENCE / FINAL TRAINER CHECK

# Regulatory / source reference

Primary reference: 29 CFR 1926.503 / applicable walking-working surface rules

https://www.osha.gov/laws-regs/regulations/standardnumber/1926/1926.503

## Before delivery

Confirm current OSHA and applicable state-plan requirements

Confirm manufacturer instructions and employer procedures

Insert workplace-specific hazards, controls, emergency contacts and rules

Use competent/qualified trainers or evaluators where required

Complete practical/workplace training and authorization requirements where applicable

Retain training records according to employer and regulatory requirements

FINAL REMINDER General fall-hazard instruction. Employer training must address the fall hazards and protection systems used in the actual work area and applicable industry standard.