TRAVISTILLEY.COM / TRAINING CENTER Powered Industrial Trucks / Forklift Fundamentals INSTRUCTOR + PRINT PACKET

| AUDIENCE | EST. TIME | REFERENCE |
| --- | --- | --- |
| Forklift / PIT operators | 40-55 min | 29 CFR 1910.178(l) |

IMPORTANT USE NOTE Formal instruction component only. Practical training and workplace evaluation by a qualified trainer are required before independent operation.

# What is inside this packet

Instructor quick-start and site-specific preparation checklist

Slide-by-slide trainer guide with discussion prompts

Employee handout / key takeaways

Printable 10-question knowledge check

Separate answer key and coaching notes

Attendance record

Practical / workplace evaluation checklist

Certificate wording and completion record

Primary regulatory/source reference

INSTRUCTOR QUICK-START

# Run a strong session without overcomplicating it.

| 1. PREPARE | Review the deck and packet. Insert your procedures, equipment, routes, photos, emergency contacts and site rules. |
| --- | --- |
| 2. PRESENT | Use the PowerPoint/PDF on a screen. Pause for discussion rather than reading every bullet. |
| 3. CHECK | Give the knowledge check. Review missed items and coach to the expected decision. |
| 4. VERIFY | Complete required hands-on or workplace-specific practice/evaluation with an appropriate trainer/evaluator. |
| 5. DOCUMENT | Use the attendance record, evaluation and certificate only after required training elements are complete. |

BEFORE EMPLOYEES WORK Formal instruction component only. Practical training and workplace evaluation by a qualified trainer are required before independent operation.

## Learning objectives

Explain forklift stability, load center and center-of-gravity basics

Use safe travel, visibility, pedestrian and dock practices

Recognize truck-specific and workplace-specific hazards

Understand OSHA requirements for practical training, evaluation, refresher training and certification

TRAINER GUIDE / MODULES 1-2

# 1. Forklifts are not cars

Rear-wheel steering changes how the back end swings.

A high center of gravity and narrow stability triangle make tip-over possible.

Loads change steering, braking, visibility and stability.

DISCUSSION PROMPT Where could this fail here?

# 2. Pre-use inspection

Inspect tires, forks, mast, chains, hydraulics, controls, horn, brakes, lights and safety devices as applicable.

Remove unsafe trucks from service.

Know battery/LP/fuel hazards and charging/refueling rules.

DISCUSSION PROMPT What control do we use?

TRAINER GUIDE / MODULES 3-4

# 3. Load handling

Know capacity plate limits and attachments.

Center and secure the load; tilt only as needed.

Travel with the load low enough for stability and visibility.

Never raise or transport people on forks unless using an approved personnel platform and procedure.

DISCUSSION PROMPT What would make you stop?

# 4. Travel and visibility

Look in the direction of travel; travel in reverse when the load blocks the view unless site rules specify another safe method.

Slow at intersections, blind corners and wet/uneven surfaces.

Maintain clearances from edges, racks, doorways and overhead hazards.

DISCUSSION PROMPT Where could this fail here?

TRAINER GUIDE / MODULES 5-6

# 5. Pedestrians, docks and trailers

Separate pedestrian routes where feasible.

Sound the horn at cross aisles and obstructed locations per site rules.

Verify dock plates, trailer restraint/chocks and floor capacity before entry.

Never allow people under elevated forks or loads.

DISCUSSION PROMPT What control do we use?

# 6. Training and evaluation

Formal instruction must be combined with practical training and workplace evaluation.

Operators need refresher training after unsafe operation, an accident/near miss, unsafe evaluation, different truck assignment, or workplace change.

Operator performance must be evaluated at least once every three years.

DISCUSSION PROMPT What would make you stop?

FACILITATED DISCUSSION

# Stop-work / red-flag conditions

Capacity plate is unreadable or attachment changes capacity

Load blocks view and operator continues forward blindly

Pedestrian enters the truck path without eye contact or separation

Truck has brake, steering, fork, tire or hydraulic defects

SCENARIO A loaded forklift is approaching a blind intersection while a pedestrian is walking in the cross aisle. What should the operator do?

RECOMMENDED RESPONSE Slow or stop as needed, sound the horn per site rules, maintain control and proceed only after the path is clear and pedestrian separation is established.

## Make it site-specific

Train on each truck type actually used and manufacturer instructions

Review traffic routes, pedestrian controls, ramps, docks, trailers and hazardous areas

Conduct hands-on practice under qualified supervision

Evaluate the operator in the actual workplace and document training/evaluation

EMPLOYEE HANDOUT

# Powered Industrial Trucks / Forklift Fundamentals

KEEP THIS PAGE

## Key decisions to remember

Forklifts are not cars: Rear-wheel steering changes how the back end swings.

Pre-use inspection: Inspect tires, forks, mast, chains, hydraulics, controls, horn, brakes, lights and safety devices as applicable.

Load handling: Know capacity plate limits and attachments.

Travel and visibility: Look in the direction of travel; travel in reverse when the load blocks the view unless site rules specify another safe method.

Pedestrians, docks and trailers: Separate pedestrian routes where feasible.

Training and evaluation: Formal instruction must be combined with practical training and workplace evaluation.

## Stop and get help when...

Capacity plate is unreadable or attachment changes capacity

Load blocks view and operator continues forward blindly

Pedestrian enters the truck path without eye contact or separation

Truck has brake, steering, fork, tire or hydraulic defects

## Our site-specific information

☐ Train on each truck type actually used and manufacturer instructions

☐ Review traffic routes, pedestrian controls, ramps, docks, trailers and hazardous areas

☐ Conduct hands-on practice under qualified supervision

☐ Evaluate the operator in the actual workplace and document training/evaluation

Employee notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

EMPLOYEE KNOWLEDGE CHECK

# Powered Industrial Trucks / Forklift Fundamentals - Knowledge Check

Employee: ____________________________________________      Date: ____________________

1. OSHA PIT training must include:

☐ A. Formal instruction, practical training and workplace evaluation

☐ B. Online training only

☐ C. A written test only

☐ D. One year of driving experience

2. Operator performance must be evaluated at least:

☐ A. Once every three years

☐ B. Every ten years

☐ C. Only after an accident

☐ D. Never if training is documented

3. A forklift load should generally be carried:

☐ A. Low and stable while traveling

☐ B. As high as possible for visibility

☐ C. At shoulder height

☐ D. With the mast fully forward

4. If a load blocks forward view, the operator should:

☐ A. Use a safe travel method such as reverse travel when appropriate and allowed

☐ B. Drive forward quickly

☐ C. Raise the load higher

☐ D. Ask a pedestrian to walk in front

5. Pedestrians should:

☐ A. Be kept clear of the truck/load path and never stand under raised forks

☐ B. Walk under raised forks if the truck is stopped

☐ C. Ride on the counterweight

☐ D. Guide loads by holding the forks

Score: ______ / 10     Passing requirement / retraining criteria: ______________________________________________

EMPLOYEE KNOWLEDGE CHECK / CONTINUED

6. A truck with a safety defect should be:

☐ A. Removed from service until corrected

☐ B. Used only on day shift

☐ C. Driven slower

☐ D. Marked with tape and used

7. Refresher training can be triggered by:

☐ A. Unsafe operation, accident/near miss, unsafe evaluation, different truck or workplace change

☐ B. Only annual calendar date

☐ C. Only a new supervisor

☐ D. Only operator request

8. The capacity plate matters because it:

☐ A. Shows rated capacity and conditions/attachments that affect capacity

☐ B. Shows employee certification

☐ C. Replaces the pre-use inspection

☐ D. Sets the speed limit

9. At a blind intersection the operator should:

☐ A. Slow and use the horn/other site controls

☐ B. Accelerate through quickly

☐ C. Raise the load

☐ D. Turn off warning devices

10. Passing this course by itself means the employee is OSHA-certified to operate a forklift.

☐ A. False

☐ B. True

Employee signature: _____________________________________________

INSTRUCTOR ONLY

# Answer key + coaching notes

1. A - Formal instruction, practical training and workplace evaluation  |  OSHA PIT training must include:

2. A - Once every three years  |  Operator performance must be evaluated at least:

3. A - Low and stable while traveling  |  A forklift load should generally be carried:

4. A - Use a safe travel method such as reverse travel when appropriate and allowed  |  If a load blocks forward view, the operator should:

5. A - Be kept clear of the truck/load path and never stand under raised forks  |  Pedestrians should:

6. A - Removed from service until corrected  |  A truck with a safety defect should be:

7. A - Unsafe operation, accident/near miss, unsafe evaluation, different truck or workplace change  |  Refresher training can be triggered by:

8. A - Shows rated capacity and conditions/attachments that affect capacity  |  The capacity plate matters because it:

9. A - Slow and use the horn/other site controls  |  At a blind intersection the operator should:

10. A - False  |  Passing this course by itself means the employee is OSHA-certified to operate a forklift.

COACHING RULE Do not treat a passing written score as proof of hands-on competency. Retrain missed critical concepts and complete workplace-specific verification where required.

TRAINING RECORD

# Attendance + completion record

Course: Powered Industrial Trucks / Forklift Fundamentals

Date: ____________   Instructor: ______________________________   Department / Location: ______________________________

| Employee Printed Name | Employee Signature | Job / Department | Date |
| --- | --- | --- | --- |

Site-specific topics / notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

PRACTICAL / WORKPLACE VERIFICATION

# Can the employee apply it?

Employee: ______________________________   Evaluator: ______________________________   Date: ____________

| Evaluation Item | Meets | Coach | N/A |
| --- | --- | --- | --- |
| Completes pre-use inspection | ☐ | ☐ | ☐ |
| Uses seat belt/restraint when provided | ☐ | ☐ | ☐ |
| Maintains low stable travel position | ☐ | ☐ | ☐ |
| Uses safe speed and controlled turns | ☐ | ☐ | ☐ |
| Demonstrates safe load pickup/placement | ☐ | ☐ | ☐ |
| Maintains pedestrian separation and horn use | ☐ | ☐ | ☐ |
| Safely approaches docks/ramps/trailers | ☐ | ☐ | ☐ |
| Parks truck with forks lowered and controls neutralized | ☐ | ☐ | ☐ |

Comments / coaching:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Result:  ☐ Competent for evaluated task (subject to employer authorization)   ☐ More training/practice needed

Evaluator signature: ______________________________   Employee signature: ______________________________

COMPLETION CERTIFICATE

CERTIFICATE OF TRAINING

This certifies that
____________________________________________

completed the formal / awareness training portion of
Powered Industrial Trucks / Forklift Fundamentals

Date: ____________________     Instructor: ____________________
Employer / Location: __________________________________________

Formal instruction component only. Practical training and workplace evaluation by a qualified trainer are required before independent operation.

REFERENCE / FINAL TRAINER CHECK

# Regulatory / source reference

Primary reference: 29 CFR 1910.178(l)

https://www.osha.gov/etools/powered-industrial-trucks/training

## Before delivery

Confirm current OSHA and applicable state-plan requirements

Confirm manufacturer instructions and employer procedures

Insert workplace-specific hazards, controls, emergency contacts and rules

Use competent/qualified trainers or evaluators where required

Complete practical/workplace training and authorization requirements where applicable

Retain training records according to employer and regulatory requirements

FINAL REMINDER Formal instruction component only. Practical training and workplace evaluation by a qualified trainer are required before independent operation.