TRAVISTILLEY.COM / TRAINING CENTER Front-End Loader Safety Fundamentals INSTRUCTOR + PRINT PACKET

| AUDIENCE | EST. TIME | REFERENCE |
| --- | --- | --- |
| Loader operators & spotters | 30-40 min | 29 CFR 1926.602 / applicable ROPS requirements + manufacturer instructions |

IMPORTANT USE NOTE General formal instruction. Employer must provide model-specific hands-on training, manufacturer requirements and task/worksite evaluation.

# What is inside this packet

Instructor quick-start and site-specific preparation checklist

Slide-by-slide trainer guide with discussion prompts

Employee handout / key takeaways

Printable 10-question knowledge check

Separate answer key and coaching notes

Attendance record

Practical / workplace evaluation checklist

Certificate wording and completion record

Primary regulatory/source reference

INSTRUCTOR QUICK-START

# Run a strong session without overcomplicating it.

| 1. PREPARE | Review the deck and packet. Insert your procedures, equipment, routes, photos, emergency contacts and site rules. |
| --- | --- |
| 2. PRESENT | Use the PowerPoint/PDF on a screen. Pause for discussion rather than reading every bullet. |
| 3. CHECK | Give the knowledge check. Review missed items and coach to the expected decision. |
| 4. VERIFY | Complete required hands-on or workplace-specific practice/evaluation with an appropriate trainer/evaluator. |
| 5. DOCUMENT | Use the attendance record, evaluation and certificate only after required training elements are complete. |

BEFORE EMPLOYEES WORK General formal instruction. Employer must provide model-specific hands-on training, manufacturer requirements and task/worksite evaluation.

## Learning objectives

Recognize rollover, struck-by, blind-spot and load hazards

Inspect loader, attachment and work area before use

Travel, load, dump and park with stable bucket position and controlled speed

Coordinate safely with spotters, pedestrians, trucks and slopes

TRAINER GUIDE / MODULES 1-2

# 1. Machine stability

Bucket/load changes the center of gravity.

Slopes, uneven ground, sharp turns, raised loads and speed increase rollover risk.

Use ROPS and seat belt/restraint as equipped and required.

DISCUSSION PROMPT Where could this fail here?

# 2. Pre-use inspection

Check tires/tracks, steering, brakes, hydraulics, bucket/attachment, pins, lights, horn/back-up alarm/camera and safety devices.

Walk the area for drop-offs, soft ground, overhead lines and traffic.

DISCUSSION PROMPT What control do we use?

TRAINER GUIDE / MODULES 3-4

# 3. Visibility and people

Large blind areas exist around loaders.

Use cameras/mirrors as aids, not substitutes for looking and site controls.

Use a trained spotter when required and establish clear signals.

DISCUSSION PROMPT What would make you stop?

# 4. Travel and load position

Carry the bucket/load low for stability and visibility.

Avoid sudden turns/braking and travel at a speed suitable for ground conditions.

Do not travel with people in the bucket.

DISCUSSION PROMPT Where could this fail here?

TRAINER GUIDE / MODULES 5-6

# 5. Loading and dumping

Approach piles/trucks squarely and maintain stable ground.

Do not undercut unstable material or work below unsupported loads.

Keep people clear of bucket swing/dump zones.

DISCUSSION PROMPT What control do we use?

# 6. Slopes, parking and maintenance

Follow manufacturer slope limits and travel orientation.

Park on stable ground, lower attachment, neutralize controls, set brake and shut down.

Block/support raised components before maintenance; never rely only on hydraulics.

DISCUSSION PROMPT What would make you stop?

FACILITATED DISCUSSION

# Stop-work / red-flag conditions

Traveling with bucket high

Person enters blind spot without positive communication

Working near edge/soft ground or excessive slope

Maintenance under a raised bucket without approved blocking/support

SCENARIO A loader operator needs to drive across uneven ground with a full bucket. What bucket position is generally safest?

RECOMMENDED RESPONSE Carry the load as low as practical while maintaining safe ground clearance, use controlled speed and follow the manufacturer/site procedure.

## Make it site-specific

Actual loader controls, attachment and manufacturer manual

Site traffic plan, spotter signals and backing controls

Slope/edge/ground conditions and overhead clearances

Hands-on loading/dumping and workplace evaluation

EMPLOYEE HANDOUT

# Front-End Loader Safety Fundamentals

KEEP THIS PAGE

## Key decisions to remember

Machine stability: Bucket/load changes the center of gravity.

Pre-use inspection: Check tires/tracks, steering, brakes, hydraulics, bucket/attachment, pins, lights, horn/back-up alarm/camera and safety devices.

Visibility and people: Large blind areas exist around loaders.

Travel and load position: Carry the bucket/load low for stability and visibility.

Loading and dumping: Approach piles/trucks squarely and maintain stable ground.

Slopes, parking and maintenance: Follow manufacturer slope limits and travel orientation.

## Stop and get help when...

Traveling with bucket high

Person enters blind spot without positive communication

Working near edge/soft ground or excessive slope

Maintenance under a raised bucket without approved blocking/support

## Our site-specific information

☐ Actual loader controls, attachment and manufacturer manual

☐ Site traffic plan, spotter signals and backing controls

☐ Slope/edge/ground conditions and overhead clearances

☐ Hands-on loading/dumping and workplace evaluation

Employee notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

EMPLOYEE KNOWLEDGE CHECK

# Front-End Loader Safety Fundamentals - Knowledge Check

Employee: ____________________________________________      Date: ____________________

1. A raised loader bucket generally:

☐ A. Raises the center of gravity and can reduce stability

☐ B. Improves stability

☐ C. Eliminates blind spots

☐ D. Increases brake capacity

2. A loader operator should normally travel with the bucket:

☐ A. Low as practical for stability/visibility

☐ B. Fully raised

☐ C. Dragging on the ground

☐ D. Above the cab

3. Blind spots are best controlled with:

☐ A. Site controls, operator checks and spotters/cameras as appropriate

☐ B. Speed alone

☐ C. Horn disabled

☐ D. Only mirrors

4. Before operating, inspect:

☐ A. Machine and work area

☐ B. Only fuel level

☐ C. Only bucket teeth

☐ D. Only paperwork

5. On slopes the operator should:

☐ A. Follow manufacturer/site limits and procedures

☐ B. Travel any direction at full speed

☐ C. Keep bucket raised

☐ D. Turn sharply

Score: ______ / 10     Passing requirement / retraining criteria: ______________________________________________

EMPLOYEE KNOWLEDGE CHECK / CONTINUED

6. People should be:

☐ A. Kept clear of bucket/load and machine travel zones

☐ B. Allowed to ride in the bucket

☐ C. Used as wheel chocks

☐ D. Positioned under raised loads

7. Maintenance under a raised attachment requires:

☐ A. Approved blocking/support and energy control as applicable

☐ B. Hydraulic pressure only

☐ C. Engine running

☐ D. A warning sign only

8. ROPS and operator restraint:

☐ A. Work together to protect the operator in a rollover when used as designed

☐ B. Are optional decorations

☐ C. Eliminate the need for training

☐ D. Allow higher travel speed

9. A spotter should use:

☐ A. Agreed signals and remain visible/safely positioned

☐ B. Random hand motions

☐ C. A position directly behind the loader

☐ D. No communication

10. This course alone qualifies an operator on every loader model.

☐ A. False

☐ B. True

Employee signature: _____________________________________________

INSTRUCTOR ONLY

# Answer key + coaching notes

1. A - Raises the center of gravity and can reduce stability  |  A raised loader bucket generally:

2. A - Low as practical for stability/visibility  |  A loader operator should normally travel with the bucket:

3. A - Site controls, operator checks and spotters/cameras as appropriate  |  Blind spots are best controlled with:

4. A - Machine and work area  |  Before operating, inspect:

5. A - Follow manufacturer/site limits and procedures  |  On slopes the operator should:

6. A - Kept clear of bucket/load and machine travel zones  |  People should be:

7. A - Approved blocking/support and energy control as applicable  |  Maintenance under a raised attachment requires:

8. A - Work together to protect the operator in a rollover when used as designed  |  ROPS and operator restraint:

9. A - Agreed signals and remain visible/safely positioned  |  A spotter should use:

10. A - False  |  This course alone qualifies an operator on every loader model.

COACHING RULE Do not treat a passing written score as proof of hands-on competency. Retrain missed critical concepts and complete workplace-specific verification where required.

TRAINING RECORD

# Attendance + completion record

Course: Front-End Loader Safety Fundamentals

Date: ____________   Instructor: ______________________________   Department / Location: ______________________________

| Employee Printed Name | Employee Signature | Job / Department | Date |
| --- | --- | --- | --- |

Site-specific topics / notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

PRACTICAL / WORKPLACE VERIFICATION

# Can the employee apply it?

Employee: ______________________________   Evaluator: ______________________________   Date: ____________

| Evaluation Item | Meets | Coach | N/A |
| --- | --- | --- | --- |
| Performs walk-around/pre-use inspection | ☐ | ☐ | ☐ |
| Uses seat belt/restraint | ☐ | ☐ | ☐ |
| Checks blind areas and work zone | ☐ | ☐ | ☐ |
| Maintains low travel load | ☐ | ☐ | ☐ |
| Controls speed/turns | ☐ | ☐ | ☐ |
| Coordinates with spotter | ☐ | ☐ | ☐ |
| Loads/dumps without exposing people | ☐ | ☐ | ☐ |
| Parks/lowers attachment safely | ☐ | ☐ | ☐ |

Comments / coaching:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Result:  ☐ Competent for evaluated task (subject to employer authorization)   ☐ More training/practice needed

Evaluator signature: ______________________________   Employee signature: ______________________________

COMPLETION CERTIFICATE

CERTIFICATE OF TRAINING

This certifies that
____________________________________________

completed the formal / awareness training portion of
Front-End Loader Safety Fundamentals

Date: ____________________     Instructor: ____________________
Employer / Location: __________________________________________

General formal instruction. Employer must provide model-specific hands-on training, manufacturer requirements and task/worksite evaluation.

REFERENCE / FINAL TRAINER CHECK

# Regulatory / source reference

Primary reference: 29 CFR 1926.602 / applicable ROPS requirements + manufacturer instructions

https://www.osha.gov/laws-regs/regulations/standardnumber/1926/1926.602

## Before delivery

Confirm current OSHA and applicable state-plan requirements

Confirm manufacturer instructions and employer procedures

Insert workplace-specific hazards, controls, emergency contacts and rules

Use competent/qualified trainers or evaluators where required

Complete practical/workplace training and authorization requirements where applicable

Retain training records according to employer and regulatory requirements

FINAL REMINDER General formal instruction. Employer must provide model-specific hands-on training, manufacturer requirements and task/worksite evaluation.