TRAVISTILLEY.COM / TRAINING CENTER Hazard Communication / GHS INSTRUCTOR + PRINT PACKET

| AUDIENCE | EST. TIME | REFERENCE |
| --- | --- | --- |
| Employees exposed to hazardous chemicals | 35-45 min | 29 CFR 1910.1200 |

IMPORTANT USE NOTE General HazCom instruction. Employer training must address the chemicals/hazards in the employee work area and the location/accessibility of the written program and SDSs.

# What is inside this packet

Instructor quick-start and site-specific preparation checklist

Slide-by-slide trainer guide with discussion prompts

Employee handout / key takeaways

Printable 10-question knowledge check

Separate answer key and coaching notes

Attendance record

Practical / workplace evaluation checklist

Certificate wording and completion record

Primary regulatory/source reference

INSTRUCTOR QUICK-START

# Run a strong session without overcomplicating it.

| 1. PREPARE | Review the deck and packet. Insert your procedures, equipment, routes, photos, emergency contacts and site rules. |
| --- | --- |
| 2. PRESENT | Use the PowerPoint/PDF on a screen. Pause for discussion rather than reading every bullet. |
| 3. CHECK | Give the knowledge check. Review missed items and coach to the expected decision. |
| 4. VERIFY | Complete required hands-on or workplace-specific practice/evaluation with an appropriate trainer/evaluator. |
| 5. DOCUMENT | Use the attendance record, evaluation and certificate only after required training elements are complete. |

BEFORE EMPLOYEES WORK General HazCom instruction. Employer training must address the chemicals/hazards in the employee work area and the location/accessibility of the written program and SDSs.

## Learning objectives

Use labels, pictograms, signal words and hazard statements

Navigate a 16-section Safety Data Sheet

Recognize exposure routes, protective measures and emergency actions

Know where the employer HazCom program, chemical list and SDSs are located

TRAINER GUIDE / MODULES 1-2

# 1. HazCom is an information system

The program connects chemical classification, labels, SDSs and employee training.

Employees need information at initial assignment and when new chemical hazards are introduced.

DISCUSSION PROMPT Where could this fail here?

# 2. Reading a shipped-container label

Product identifier links the label to the SDS.

Signal word indicates relative severity.

Pictograms, hazard statements and precautionary statements describe hazards and controls.

DISCUSSION PROMPT What control do we use?

TRAINER GUIDE / MODULES 3-4

# 3. Workplace labels

Secondary/workplace containers need compliant identification/warning under the employer system.

Never use an unlabeled chemical container unless the employer’s limited immediate-use exception truly applies.

DISCUSSION PROMPT What would make you stop?

# 4. Safety Data Sheets

Know how to find hazard identification, first aid, fire-fighting, spill, handling/storage, exposure controls/PPE and physical properties.

SDSs must be readily accessible during the work shift.

DISCUSSION PROMPT Where could this fail here?

TRAINER GUIDE / MODULES 5-6

# 5. Exposure and controls

Understand inhalation, skin/eye contact, ingestion and injection routes.

Use engineering controls, safe work practices and PPE identified by the employer.

Know how to detect release using odor/appearance/monitoring only as appropriate.

DISCUSSION PROMPT What control do we use?

# 6. Spill and emergency response

Employees need to know what they are authorized to handle versus when to evacuate/report.

Follow site alarm, spill kit and emergency-response procedures.

Do not improvise chemical response without training and equipment.

DISCUSSION PROMPT What would make you stop?

FACILITATED DISCUSSION

# Stop-work / red-flag conditions

Unlabeled or unknown container

SDS cannot be accessed during the shift

Chemical transferred to an incompatible container

Employee does not know the emergency reporting/evacuation action

SCENARIO A worker finds a spray bottle with clear liquid and no label. A coworker says, “It is probably glass cleaner.” What should happen?

RECOMMENDED RESPONSE Do not use the unknown chemical. Isolate/report it and follow the employer procedure to identify and label or dispose of it safely.

## Make it site-specific

Show the written HazCom program and chemical inventory

Show how to access SDSs, including backup access if electronic

Review actual workplace chemicals, labels and pictograms

Cover spill/emergency procedures and task-specific PPE/controls

EMPLOYEE HANDOUT

# Hazard Communication / GHS

KEEP THIS PAGE

## Key decisions to remember

HazCom is an information system: The program connects chemical classification, labels, SDSs and employee training.

Reading a shipped-container label: Product identifier links the label to the SDS.

Workplace labels: Secondary/workplace containers need compliant identification/warning under the employer system.

Safety Data Sheets: Know how to find hazard identification, first aid, fire-fighting, spill, handling/storage, exposure controls/PPE and physical properties.

Exposure and controls: Understand inhalation, skin/eye contact, ingestion and injection routes.

Spill and emergency response: Employees need to know what they are authorized to handle versus when to evacuate/report.

## Stop and get help when...

Unlabeled or unknown container

SDS cannot be accessed during the shift

Chemical transferred to an incompatible container

Employee does not know the emergency reporting/evacuation action

## Our site-specific information

☐ Show the written HazCom program and chemical inventory

☐ Show how to access SDSs, including backup access if electronic

☐ Review actual workplace chemicals, labels and pictograms

☐ Cover spill/emergency procedures and task-specific PPE/controls

Employee notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

EMPLOYEE KNOWLEDGE CHECK

# Hazard Communication / GHS - Knowledge Check

Employee: ____________________________________________      Date: ____________________

1. HazCom communicates chemical hazards through:

☐ A. Labels, SDSs, a written program and employee training

☐ B. Color coding only

☐ C. Annual posters only

☐ D. Verbal warnings only

2. The signal word on a GHS label is typically:

☐ A. Danger or Warning

☐ B. Stop or Go

☐ C. Safe or Unsafe

☐ D. Red or Yellow

3. An SDS should be:

☐ A. Readily accessible during each work shift

☐ B. Kept only at corporate office

☐ C. Available only to supervisors

☐ D. Destroyed after training

4. A product identifier helps:

☐ A. Link the label and SDS to the chemical

☐ B. Set the purchase price

☐ C. Show employee ownership

☐ D. Determine shift assignment

5. An unlabeled mystery chemical should be:

☐ A. Not used until properly identified and handled under employer procedure

☐ B. Used carefully

☐ C. Smelled to identify it

☐ D. Mixed with water

Score: ______ / 10     Passing requirement / retraining criteria: ______________________________________________

EMPLOYEE KNOWLEDGE CHECK / CONTINUED

6. HazCom training is required when:

☐ A. Employees are initially assigned and when new chemical hazards are introduced

☐ B. Only every five years

☐ C. Only after a spill

☐ D. Only for supervisors

7. SDS Section 8 generally addresses:

☐ A. Exposure controls/personal protection

☐ B. Transportation only

☐ C. Ecological information only

☐ D. Supplier billing

8. Chemical protection should include:

☐ A. Controls and PPE appropriate to the hazard and task

☐ B. Any gloves available

☐ C. Only safety glasses

☐ D. Only ventilation if convenient

9. Employees should know:

☐ A. Where the written program, chemical list and SDSs are available

☐ B. Only the chemical brand

☐ C. Only the supplier name

☐ D. Only the pictogram shape

10. This generic course replaces training on the actual chemicals in the work area.

☐ A. False

☐ B. True

Employee signature: _____________________________________________

INSTRUCTOR ONLY

# Answer key + coaching notes

1. A - Labels, SDSs, a written program and employee training  |  HazCom communicates chemical hazards through:

2. A - Danger or Warning  |  The signal word on a GHS label is typically:

3. A - Readily accessible during each work shift  |  An SDS should be:

4. A - Link the label and SDS to the chemical  |  A product identifier helps:

5. A - Not used until properly identified and handled under employer procedure  |  An unlabeled mystery chemical should be:

6. A - Employees are initially assigned and when new chemical hazards are introduced  |  HazCom training is required when:

7. A - Exposure controls/personal protection  |  SDS Section 8 generally addresses:

8. A - Controls and PPE appropriate to the hazard and task  |  Chemical protection should include:

9. A - Where the written program, chemical list and SDSs are available  |  Employees should know:

10. A - False  |  This generic course replaces training on the actual chemicals in the work area.

COACHING RULE Do not treat a passing written score as proof of hands-on competency. Retrain missed critical concepts and complete workplace-specific verification where required.

TRAINING RECORD

# Attendance + completion record

Course: Hazard Communication / GHS

Date: ____________   Instructor: ______________________________   Department / Location: ______________________________

| Employee Printed Name | Employee Signature | Job / Department | Date |
| --- | --- | --- | --- |

Site-specific topics / notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

PRACTICAL / WORKPLACE VERIFICATION

# Can the employee apply it?

Employee: ______________________________   Evaluator: ______________________________   Date: ____________

| Evaluation Item | Meets | Coach | N/A |
| --- | --- | --- | --- |
| Finds an SDS for a workplace chemical | ☐ | ☐ | ☐ |
| Identifies product identifier/signal word/pictograms | ☐ | ☐ | ☐ |
| Finds first-aid and PPE information on SDS | ☐ | ☐ | ☐ |
| Explains secondary-container labeling | ☐ | ☐ | ☐ |
| Describes site spill/reporting action | ☐ | ☐ | ☐ |
| Identifies required PPE/controls for example task | ☐ | ☐ | ☐ |

Comments / coaching:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Result:  ☐ Competent for evaluated task (subject to employer authorization)   ☐ More training/practice needed

Evaluator signature: ______________________________   Employee signature: ______________________________

COMPLETION CERTIFICATE

CERTIFICATE OF TRAINING

This certifies that
____________________________________________

completed the formal / awareness training portion of
Hazard Communication / GHS

Date: ____________________     Instructor: ____________________
Employer / Location: __________________________________________

General HazCom instruction. Employer training must address the chemicals/hazards in the employee work area and the location/accessibility of the written program and SDSs.

REFERENCE / FINAL TRAINER CHECK

# Regulatory / source reference

Primary reference: 29 CFR 1910.1200

https://www.osha.gov/laws-regs/regulations/standardnumber/1910/1910.1200

## Before delivery

Confirm current OSHA and applicable state-plan requirements

Confirm manufacturer instructions and employer procedures

Insert workplace-specific hazards, controls, emergency contacts and rules

Use competent/qualified trainers or evaluators where required

Complete practical/workplace training and authorization requirements where applicable

Retain training records according to employer and regulatory requirements

FINAL REMINDER General HazCom instruction. Employer training must address the chemicals/hazards in the employee work area and the location/accessibility of the written program and SDSs.