TRAVISTILLEY.COM / TRAINING CENTER Lockout / Tagout (LOTO) INSTRUCTOR + PRINT PACKET

| AUDIENCE | EST. TIME | REFERENCE |
| --- | --- | --- |
| Authorized & affected employees | 35-45 min | 29 CFR 1910.147 |

IMPORTANT USE NOTE General/formal instruction. Employer- and machine-specific energy-control procedures are still required.

# What is inside this packet

Instructor quick-start and site-specific preparation checklist

Slide-by-slide trainer guide with discussion prompts

Employee handout / key takeaways

Printable 10-question knowledge check

Separate answer key and coaching notes

Attendance record

Practical / workplace evaluation checklist

Certificate wording and completion record

Primary regulatory/source reference

INSTRUCTOR QUICK-START

# Run a strong session without overcomplicating it.

| 1. PREPARE | Review the deck and packet. Insert your procedures, equipment, routes, photos, emergency contacts and site rules. |
| --- | --- |
| 2. PRESENT | Use the PowerPoint/PDF on a screen. Pause for discussion rather than reading every bullet. |
| 3. CHECK | Give the knowledge check. Review missed items and coach to the expected decision. |
| 4. VERIFY | Complete required hands-on or workplace-specific practice/evaluation with an appropriate trainer/evaluator. |
| 5. DOCUMENT | Use the attendance record, evaluation and certificate only after required training elements are complete. |

BEFORE EMPLOYEES WORK General/formal instruction. Employer- and machine-specific energy-control procedures are still required.

## Learning objectives

Recognize hazardous energy sources and when LOTO applies

Explain authorized, affected and other employee roles

Follow the basic sequence for shutdown, isolation, lock/tag, stored-energy control and verification

Recognize group, shift-change and contractor situations that need special control

TRAINER GUIDE / MODULES 1-2

# 1. When LOTO applies

Servicing or maintenance can expose workers to unexpected startup, movement, pressure, gravity, heat or stored energy.

A normal stop button is not an energy-isolating device.

Production work can become covered when guards are bypassed or a body part enters a danger zone.

DISCUSSION PROMPT Where could this fail here?

# 2. Roles and responsibilities

Authorized employees apply and control locks/tags for servicing work.

Affected employees operate/use the equipment or work in the area and must understand the purpose of the procedure.

Other employees must know not to restart or interfere with locked/tagged equipment.

DISCUSSION PROMPT What control do we use?

TRAINER GUIDE / MODULES 3-4

# 3. The energy-control sequence

Prepare for shutdown and identify every energy source.

Notify affected employees, stop the machine, isolate energy sources and apply locks/tags.

Release, block or restrain stored energy.

Verify isolation before work begins.

DISCUSSION PROMPT What would make you stop?

# 4. Verification is a step, not a feeling

Try-start or other approved verification must confirm the isolation is effective.

Testing must be performed using the employer procedure and appropriate instruments where needed.

Re-verify when conditions change or energy could reaccumulate.

DISCUSSION PROMPT Where could this fail here?

TRAINER GUIDE / MODULES 5-6

# 5. Group work and shift changes

Each exposed authorized employee needs equivalent personal protection.

Shift changes must maintain continuity of protection.

Contractors and host employers must exchange LOTO procedure information.

DISCUSSION PROMPT What control do we use?

# 6. Safe return to service

Inspect the work area, replace guards and remove tools.

Confirm employees are clear and account for personnel.

Locks/tags are normally removed by the employee who applied them, except under a documented employer procedure.

Notify affected employees before returning equipment to service.

DISCUSSION PROMPT What would make you stop?

FACILITATED DISCUSSION

# Stop-work / red-flag conditions

Someone says “it is off” but cannot identify all energy sources

A lock is missing a name/identity

Stored pressure, gravity or heat has not been controlled

Someone wants to remove another employee’s lock without the employer procedure

SCENARIO A mechanic locks out electrical power to a conveyor but does not block the raised pneumatic gate. The gate can fall if pressure bleeds off. What is missing?

RECOMMENDED RESPONSE Stored energy must be relieved, restrained or otherwise rendered safe before work begins.

## Make it site-specific

Show the written energy-control program and machine-specific procedures

Identify actual isolation points and stored-energy hazards

Demonstrate employer locks/tags and verification method

Explain group LOTO, contractor coordination, shift change and emergency lock-removal rules

EMPLOYEE HANDOUT

# Lockout / Tagout (LOTO)

KEEP THIS PAGE

## Key decisions to remember

When LOTO applies: Servicing or maintenance can expose workers to unexpected startup, movement, pressure, gravity, heat or stored energy.

Roles and responsibilities: Authorized employees apply and control locks/tags for servicing work.

The energy-control sequence: Prepare for shutdown and identify every energy source.

Verification is a step, not a feeling: Try-start or other approved verification must confirm the isolation is effective.

Group work and shift changes: Each exposed authorized employee needs equivalent personal protection.

Safe return to service: Inspect the work area, replace guards and remove tools.

## Stop and get help when...

Someone says “it is off” but cannot identify all energy sources

A lock is missing a name/identity

Stored pressure, gravity or heat has not been controlled

Someone wants to remove another employee’s lock without the employer procedure

## Our site-specific information

☐ Show the written energy-control program and machine-specific procedures

☐ Identify actual isolation points and stored-energy hazards

☐ Demonstrate employer locks/tags and verification method

☐ Explain group LOTO, contractor coordination, shift change and emergency lock-removal rules

Employee notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

EMPLOYEE KNOWLEDGE CHECK

# Lockout / Tagout (LOTO) - Knowledge Check

Employee: ____________________________________________      Date: ____________________

1. The main purpose of LOTO is to:

☐ A. Prevent unexpected energization or release of hazardous energy

☐ B. Replace machine guarding

☐ C. Speed up maintenance

☐ D. Track attendance

2. Before servicing begins, the authorized employee must verify:

☐ A. The machine is isolated and de-energized

☐ B. The supervisor is nearby

☐ C. The production schedule is clear

☐ D. A warning sign is posted

3. Who normally applies personal lockout devices for covered servicing?

☐ A. Authorized employees performing the servicing

☐ B. Any employee in the area

☐ C. Only security personnel

☐ D. The equipment manufacturer

4. Stored energy such as pressure or gravity must be:

☐ A. Relieved, restrained or otherwise rendered safe

☐ B. Ignored after electrical power is off

☐ C. Documented only

☐ D. Handled after the work

5. A stop button is:

☐ A. Not a substitute for energy isolation

☐ B. Always an energy isolating device

☐ C. Enough if the machine stops moving

☐ D. Equivalent to a personal lock

Score: ______ / 10     Passing requirement / retraining criteria: ______________________________________________

EMPLOYEE KNOWLEDGE CHECK / CONTINUED

6. Affected employees should be notified:

☐ A. Before controls are applied and after they are removed

☐ B. Only after the job is finished

☐ C. Only if they ask

☐ D. Only during annual training

7. If conditions allow energy to reaccumulate, workers should:

☐ A. Continue monitoring and take steps to maintain isolation

☐ B. Ignore it once the lock is applied

☐ C. Remove the lock to test it

☐ D. Ask an affected employee to watch

8. Group LOTO should provide each authorized employee:

☐ A. A level of protection equivalent to personal lockout

☐ B. One shared key controlled by production

☐ C. No personal accountability

☐ D. Only a warning tag

9. This downloadable course alone authorizes an employee to perform LOTO on any machine.

☐ A. False

☐ B. True

10. When outside servicing personnel perform covered work, host and contractor employers should:

☐ A. Inform each other of their LOTO procedures

☐ B. Use only the contractor procedure without discussion

☐ C. Skip notification if both are experienced

☐ D. Remove all employer locks

Employee signature: _____________________________________________

INSTRUCTOR ONLY

# Answer key + coaching notes

1. A - Prevent unexpected energization or release of hazardous energy  |  The main purpose of LOTO is to:

2. A - The machine is isolated and de-energized  |  Before servicing begins, the authorized employee must verify:

3. A - Authorized employees performing the servicing  |  Who normally applies personal lockout devices for covered servicing?

4. A - Relieved, restrained or otherwise rendered safe  |  Stored energy such as pressure or gravity must be:

5. A - Not a substitute for energy isolation  |  A stop button is:

6. A - Before controls are applied and after they are removed  |  Affected employees should be notified:

7. A - Continue monitoring and take steps to maintain isolation  |  If conditions allow energy to reaccumulate, workers should:

8. A - A level of protection equivalent to personal lockout  |  Group LOTO should provide each authorized employee:

9. A - False  |  This downloadable course alone authorizes an employee to perform LOTO on any machine.

10. A - Inform each other of their LOTO procedures  |  When outside servicing personnel perform covered work, host and contractor employers should:

COACHING RULE Do not treat a passing written score as proof of hands-on competency. Retrain missed critical concepts and complete workplace-specific verification where required.

TRAINING RECORD

# Attendance + completion record

Course: Lockout / Tagout (LOTO)

Date: ____________   Instructor: ______________________________   Department / Location: ______________________________

| Employee Printed Name | Employee Signature | Job / Department | Date |
| --- | --- | --- | --- |

Site-specific topics / notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

PRACTICAL / WORKPLACE VERIFICATION

# Can the employee apply it?

Employee: ______________________________   Evaluator: ______________________________   Date: ____________

| Evaluation Item | Meets | Coach | N/A |
| --- | --- | --- | --- |
| Identifies all energy sources before shutdown | ☐ | ☐ | ☐ |
| Selects correct isolation devices | ☐ | ☐ | ☐ |
| Applies personal lock/tag correctly | ☐ | ☐ | ☐ |
| Controls stored or residual energy | ☐ | ☐ | ☐ |
| Performs approved verification step | ☐ | ☐ | ☐ |
| Restores equipment using the employer sequence | ☐ | ☐ | ☐ |

Comments / coaching:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Result:  ☐ Competent for evaluated task (subject to employer authorization)   ☐ More training/practice needed

Evaluator signature: ______________________________   Employee signature: ______________________________

COMPLETION CERTIFICATE

CERTIFICATE OF TRAINING

This certifies that
____________________________________________

completed the formal / awareness training portion of
Lockout / Tagout (LOTO)

Date: ____________________     Instructor: ____________________
Employer / Location: __________________________________________

General/formal instruction. Employer- and machine-specific energy-control procedures are still required.

REFERENCE / FINAL TRAINER CHECK

# Regulatory / source reference

Primary reference: 29 CFR 1910.147

https://www.osha.gov/laws-regs/regulations/standardnumber/1910/1910.147

## Before delivery

Confirm current OSHA and applicable state-plan requirements

Confirm manufacturer instructions and employer procedures

Insert workplace-specific hazards, controls, emergency contacts and rules

Use competent/qualified trainers or evaluators where required

Complete practical/workplace training and authorization requirements where applicable

Retain training records according to employer and regulatory requirements

FINAL REMINDER General/formal instruction. Employer- and machine-specific energy-control procedures are still required.