TRAVISTILLEY.COM / TRAINING CENTER Order Picker / Stock Picker INSTRUCTOR + PRINT PACKET

| AUDIENCE | EST. TIME | REFERENCE |
| --- | --- | --- |
| Order-picker operators | 30-40 min | 29 CFR 1910.178(l) |

IMPORTANT USE NOTE Equipment-specific formal instruction. Practical training and workplace evaluation are required; fall protection must follow the truck/manufacturer and employer requirements.

# What is inside this packet

Instructor quick-start and site-specific preparation checklist

Slide-by-slide trainer guide with discussion prompts

Employee handout / key takeaways

Printable 10-question knowledge check

Separate answer key and coaching notes

Attendance record

Practical / workplace evaluation checklist

Certificate wording and completion record

Primary regulatory/source reference

INSTRUCTOR QUICK-START

# Run a strong session without overcomplicating it.

| 1. PREPARE | Review the deck and packet. Insert your procedures, equipment, routes, photos, emergency contacts and site rules. |
| --- | --- |
| 2. PRESENT | Use the PowerPoint/PDF on a screen. Pause for discussion rather than reading every bullet. |
| 3. CHECK | Give the knowledge check. Review missed items and coach to the expected decision. |
| 4. VERIFY | Complete required hands-on or workplace-specific practice/evaluation with an appropriate trainer/evaluator. |
| 5. DOCUMENT | Use the attendance record, evaluation and certificate only after required training elements are complete. |

BEFORE EMPLOYEES WORK Equipment-specific formal instruction. Practical training and workplace evaluation are required; fall protection must follow the truck/manufacturer and employer requirements.

## Learning objectives

Recognize elevated-platform, fall and crushing hazards

Use required fall-restraint/fall-protection systems correctly

Control travel and platform elevation in narrow aisles

Pick/place loads without overreach or unstable body position

TRAINER GUIDE / MODULES 1-2

# 1. The operator goes up

Order pickers elevate the operator with the load platform, creating fall and overhead exposure.

The truck must be used only as designed, with required gates/rails and fall-protection systems.

DISCUSSION PROMPT Where could this fail here?

# 2. Fall protection and body position

Use the manufacturer-required harness/restraint and approved anchorage.

Keep gates/rails in position and do not climb on them.

Do not overreach; reposition the truck/platform.

DISCUSSION PROMPT What control do we use?

TRAINER GUIDE / MODULES 3-4

# 3. Travel and elevation

Follow manufacturer/site rules for travel with the platform elevated.

Control speed and maintain clearance from rack beams, sprinklers and other overhead hazards.

Stop and lower when visibility or aisle conditions are not controlled.

DISCUSSION PROMPT What would make you stop?

# 4. Picking and load control

Stay within platform capacity and keep picked product stable.

Secure loose items and keep platform floor clear.

Use proper ergonomics and avoid twisting from unstable positions.

DISCUSSION PROMPT Where could this fail here?

TRAINER GUIDE / MODULES 5-6

# 5. Pedestrians and aisles

Control access below/around the elevated platform.

Use horns, lights, barriers or spotters as the site requires.

Never allow a person under elevated forks/platform components.

DISCUSSION PROMPT What control do we use?

# 6. Inspection and rescue readiness

Inspect lift chains/mast, controls, gates, fall-protection equipment and anchorage.

Know emergency lowering/rescue procedures and who can respond.

DISCUSSION PROMPT What would make you stop?

FACILITATED DISCUSSION

# Stop-work / red-flag conditions

Harness/lanyard/restraint is not connected as required

Gate is open or bypassed while elevated

Operator overreaches outside the platform

Travel path has overhead obstructions or people below

SCENARIO An operator cannot reach a carton from the elevated platform without leaning past the guardrail. What should they do?

RECOMMENDED RESPONSE Stop, lower/reposition the truck or platform and maintain the required body position and fall-protection system; do not overreach.

## Make it site-specific

Manufacturer-required fall protection/restraint and anchorage

Rules for elevated travel and aisle control

Emergency lowering/rescue procedure

Hands-on picking at height and workplace evaluation

EMPLOYEE HANDOUT

# Order Picker / Stock Picker

KEEP THIS PAGE

## Key decisions to remember

The operator goes up: Order pickers elevate the operator with the load platform, creating fall and overhead exposure.

Fall protection and body position: Use the manufacturer-required harness/restraint and approved anchorage.

Travel and elevation: Follow manufacturer/site rules for travel with the platform elevated.

Picking and load control: Stay within platform capacity and keep picked product stable.

Pedestrians and aisles: Control access below/around the elevated platform.

Inspection and rescue readiness: Inspect lift chains/mast, controls, gates, fall-protection equipment and anchorage.

## Stop and get help when...

Harness/lanyard/restraint is not connected as required

Gate is open or bypassed while elevated

Operator overreaches outside the platform

Travel path has overhead obstructions or people below

## Our site-specific information

☐ Manufacturer-required fall protection/restraint and anchorage

☐ Rules for elevated travel and aisle control

☐ Emergency lowering/rescue procedure

☐ Hands-on picking at height and workplace evaluation

Employee notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

EMPLOYEE KNOWLEDGE CHECK

# Order Picker / Stock Picker - Knowledge Check

Employee: ____________________________________________      Date: ____________________

1. The main difference with an order picker is that:

☐ A. The operator is elevated with the platform

☐ B. It has no forks

☐ C. It cannot enter aisles

☐ D. It has front-wheel steering only

2. When required by the truck/manufacturer, the operator should use:

☐ A. The approved fall-protection/restraint system and anchorage

☐ B. Any convenient rack beam

☐ C. A rope tied to product

☐ D. No restraint if the aisle is narrow

3. If an item is out of reach, the operator should:

☐ A. Reposition rather than overreach

☐ B. Stand on the guardrail

☐ C. Disconnect the restraint

☐ D. Lean farther out

4. People should be kept:

☐ A. Clear of the area below/around the elevated work platform

☐ B. Directly under the platform

☐ C. Inside the rack bay

☐ D. On the forks

5. Overhead hazards can include:

☐ A. Rack beams, sprinklers, pipes and structure

☐ B. Only the ceiling

☐ C. Only lights

☐ D. Only outdoor power lines

Score: ______ / 10     Passing requirement / retraining criteria: ______________________________________________

EMPLOYEE KNOWLEDGE CHECK / CONTINUED

6. Platform floor should be:

☐ A. Kept clear and loads stable

☐ B. Used to store loose debris

☐ C. Covered with oil for easier sliding

☐ D. Overloaded if needed

7. Emergency lowering procedures should be:

☐ A. Known before work at height

☐ B. Learned after a failure

☐ C. Kept only by maintenance

☐ D. Ignored if batteries are charged

8. Travel with the platform elevated should:

☐ A. Follow manufacturer and employer rules

☐ B. Always be done at full speed

☐ C. Never consider overhead clearance

☐ D. Be decided by the operator alone

9. Fall-protection equipment should be inspected:

☐ A. Before use according to the employer/manufacturer program

☐ B. Only annually

☐ C. Only after a fall

☐ D. Only by purchasing

10. This course replaces practical training/evaluation.

☐ A. False

☐ B. True

Employee signature: _____________________________________________

INSTRUCTOR ONLY

# Answer key + coaching notes

1. A - The operator is elevated with the platform  |  The main difference with an order picker is that:

2. A - The approved fall-protection/restraint system and anchorage  |  When required by the truck/manufacturer, the operator should use:

3. A - Reposition rather than overreach  |  If an item is out of reach, the operator should:

4. A - Clear of the area below/around the elevated work platform  |  People should be kept:

5. A - Rack beams, sprinklers, pipes and structure  |  Overhead hazards can include:

6. A - Kept clear and loads stable  |  Platform floor should be:

7. A - Known before work at height  |  Emergency lowering procedures should be:

8. A - Follow manufacturer and employer rules  |  Travel with the platform elevated should:

9. A - Before use according to the employer/manufacturer program  |  Fall-protection equipment should be inspected:

10. A - False  |  This course replaces practical training/evaluation.

COACHING RULE Do not treat a passing written score as proof of hands-on competency. Retrain missed critical concepts and complete workplace-specific verification where required.

TRAINING RECORD

# Attendance + completion record

Course: Order Picker / Stock Picker

Date: ____________   Instructor: ______________________________   Department / Location: ______________________________

| Employee Printed Name | Employee Signature | Job / Department | Date |
| --- | --- | --- | --- |

Site-specific topics / notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

PRACTICAL / WORKPLACE VERIFICATION

# Can the employee apply it?

Employee: ______________________________   Evaluator: ______________________________   Date: ____________

| Evaluation Item | Meets | Coach | N/A |
| --- | --- | --- | --- |
| Inspects truck and fall-protection equipment | ☐ | ☐ | ☐ |
| Connects restraint/harness correctly | ☐ | ☐ | ☐ |
| Maintains gates/rails | ☐ | ☐ | ☐ |
| Controls platform height/travel | ☐ | ☐ | ☐ |
| Maintains overhead clearances | ☐ | ☐ | ☐ |
| Uses safe reach/body position | ☐ | ☐ | ☐ |
| Controls pedestrian exposure below | ☐ | ☐ | ☐ |
| Demonstrates emergency lowering knowledge | ☐ | ☐ | ☐ |

Comments / coaching:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Result:  ☐ Competent for evaluated task (subject to employer authorization)   ☐ More training/practice needed

Evaluator signature: ______________________________   Employee signature: ______________________________

COMPLETION CERTIFICATE

CERTIFICATE OF TRAINING

This certifies that
____________________________________________

completed the formal / awareness training portion of
Order Picker / Stock Picker

Date: ____________________     Instructor: ____________________
Employer / Location: __________________________________________

Equipment-specific formal instruction. Practical training and workplace evaluation are required; fall protection must follow the truck/manufacturer and employer requirements.

REFERENCE / FINAL TRAINER CHECK

# Regulatory / source reference

Primary reference: 29 CFR 1910.178(l)

https://www.osha.gov/etools/powered-industrial-trucks/workplace/narrow-aisles

## Before delivery

Confirm current OSHA and applicable state-plan requirements

Confirm manufacturer instructions and employer procedures

Insert workplace-specific hazards, controls, emergency contacts and rules

Use competent/qualified trainers or evaluators where required

Complete practical/workplace training and authorization requirements where applicable

Retain training records according to employer and regulatory requirements

FINAL REMINDER Equipment-specific formal instruction. Practical training and workplace evaluation are required; fall protection must follow the truck/manufacturer and employer requirements.