TRAVISTILLEY.COM / TRAINING CENTER Scissor Lift Safety INSTRUCTOR + PRINT PACKET

| AUDIENCE | EST. TIME | REFERENCE |
| --- | --- | --- |
| Scissor-lift users | 30-40 min | OSHA scissor lift guidance / applicable scaffold & training requirements |

IMPORTANT USE NOTE General/formal instruction. The employer must provide equipment-specific training, manufacturer instructions and verify safe operation for the actual lift and workplace.

# What is inside this packet

Instructor quick-start and site-specific preparation checklist

Slide-by-slide trainer guide with discussion prompts

Employee handout / key takeaways

Printable 10-question knowledge check

Separate answer key and coaching notes

Attendance record

Practical / workplace evaluation checklist

Certificate wording and completion record

Primary regulatory/source reference

INSTRUCTOR QUICK-START

# Run a strong session without overcomplicating it.

| 1. PREPARE | Review the deck and packet. Insert your procedures, equipment, routes, photos, emergency contacts and site rules. |
| --- | --- |
| 2. PRESENT | Use the PowerPoint/PDF on a screen. Pause for discussion rather than reading every bullet. |
| 3. CHECK | Give the knowledge check. Review missed items and coach to the expected decision. |
| 4. VERIFY | Complete required hands-on or workplace-specific practice/evaluation with an appropriate trainer/evaluator. |
| 5. DOCUMENT | Use the attendance record, evaluation and certificate only after required training elements are complete. |

BEFORE EMPLOYEES WORK General/formal instruction. The employer must provide equipment-specific training, manufacturer instructions and verify safe operation for the actual lift and workplace.

## Learning objectives

Recognize fall, tip-over, crushing, electrical and falling-object hazards

Complete a pre-use/work-area inspection

Position, elevate, travel and lower the lift safely

Apply manufacturer, site and weather/wind restrictions

TRAINER GUIDE / MODULES 1-2

# 1. Five major hazard groups

Falls from the platform.

Tip-over/collapse from unstable setup, overload or environmental conditions.

Crushing/entrapment against overhead structures.

Electrical contact with energized conductors.

Falling objects striking people below.

DISCUSSION PROMPT Where could this fail here?

# 2. Pre-use and work-area inspection

Inspect controls, emergency lowering, guardrails/gates, tires, leaks, alarms and safety devices.

Inspect the floor/ground for holes, drop-offs, slopes, debris and weak surfaces.

Identify overhead power, beams, piping and moving equipment.

DISCUSSION PROMPT What control do we use?

TRAINER GUIDE / MODULES 3-4

# 3. Stable setup

Use only on surfaces and slopes permitted by the manufacturer.

Do not exceed platform capacity or use makeshift devices to gain height.

Do not move the lift in a way prohibited by the manufacturer when elevated.

DISCUSSION PROMPT What would make you stop?

# 4. Falls and body position

Keep gates/rails in place and maintain firm footing on the platform.

Do not climb or stand on guardrails.

Use fall protection only as required by applicable rules, manufacturer instructions and employer program; use approved anchorage only.

DISCUSSION PROMPT Where could this fail here?

TRAINER GUIDE / MODULES 5-6

# 5. Crushing and electrical

Keep adequate clearance from overhead structures and energized lines.

Use a spotter/ground guide where visibility is limited.

Move slowly in congested areas and avoid trapping the operator between platform controls and structure.

DISCUSSION PROMPT What control do we use?

# 6. Weather and emergency response

Follow manufacturer wind limits and remove the lift from service in unsafe weather.

Know emergency stop/lowering procedures.

Keep the area below controlled when falling objects are possible.

DISCUSSION PROMPT What would make you stop?

FACILITATED DISCUSSION

# Stop-work / red-flag conditions

Guardrail/gate or control is damaged

Lift is on an unapproved slope or unstable surface

Wind/environmental conditions exceed manufacturer limits

Operator is climbing on rails or working close to power/overhead structure without clearance

SCENARIO A scissor lift is fully elevated outdoors and wind begins gusting strongly. The operator is below the manufacturer’s rated platform capacity. What should happen?

RECOMMENDED RESPONSE Capacity alone does not make the situation safe. Follow the manufacturer wind limit and employer procedure; lower and discontinue use if conditions are outside safe limits.

## Make it site-specific

Actual lift model, controls, emergency lowering and manufacturer limits

Indoor/outdoor surface and wind rules

Electrical clearance and overhead-crush controls

Site traffic/pedestrian barricades and rescue response

EMPLOYEE HANDOUT

# Scissor Lift Safety

KEEP THIS PAGE

## Key decisions to remember

Five major hazard groups: Falls from the platform.

Pre-use and work-area inspection: Inspect controls, emergency lowering, guardrails/gates, tires, leaks, alarms and safety devices.

Stable setup: Use only on surfaces and slopes permitted by the manufacturer.

Falls and body position: Keep gates/rails in place and maintain firm footing on the platform.

Crushing and electrical: Keep adequate clearance from overhead structures and energized lines.

Weather and emergency response: Follow manufacturer wind limits and remove the lift from service in unsafe weather.

## Stop and get help when...

Guardrail/gate or control is damaged

Lift is on an unapproved slope or unstable surface

Wind/environmental conditions exceed manufacturer limits

Operator is climbing on rails or working close to power/overhead structure without clearance

## Our site-specific information

☐ Actual lift model, controls, emergency lowering and manufacturer limits

☐ Indoor/outdoor surface and wind rules

☐ Electrical clearance and overhead-crush controls

☐ Site traffic/pedestrian barricades and rescue response

Employee notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

EMPLOYEE KNOWLEDGE CHECK

# Scissor Lift Safety - Knowledge Check

Employee: ____________________________________________      Date: ____________________

1. A scissor lift can tip because of:

☐ A. Unstable surfaces, slopes, overload or environmental conditions

☐ B. Only electrical failure

☐ C. Only flat tires

☐ D. Only low battery

2. The platform guardrails are intended to:

☐ A. Help prevent falls and must not be used as steps

☐ B. Serve as ladders

☐ C. Support extra work platforms

☐ D. Hold tools outside the platform

3. Before elevating, the operator should inspect:

☐ A. The lift and the work area, including overhead hazards

☐ B. Only battery charge

☐ C. Only the weather app

☐ D. Only the floor inside the platform

4. Standing on the guardrail to gain extra height is:

☐ A. Unsafe and not permitted

☐ B. Allowed if another worker holds the rail

☐ C. Allowed indoors

☐ D. Required for ceiling work

5. Wind limits should come from:

☐ A. Manufacturer instructions and employer procedure

☐ B. Operator guess

☐ C. The load weight only

☐ D. A fixed universal mph for every lift

Score: ______ / 10     Passing requirement / retraining criteria: ______________________________________________

EMPLOYEE KNOWLEDGE CHECK / CONTINUED

6. Overhead crushing hazards require:

☐ A. Clearance, slow control and often a spotter/site controls

☐ B. Faster platform movement

☐ C. Leaning outside the platform

☐ D. Disabling upper controls

7. Electrical hazards should be controlled by:

☐ A. Maintaining required clearance and following site/manufacturer rules

☐ B. Relying on tires as insulation

☐ C. Touching the line with a tool first

☐ D. Using wet gloves

8. People below the lift should:

☐ A. Be protected from falling objects and equipment movement

☐ B. Stand under the platform

☐ C. Hold the rails

☐ D. Guide the lift by hand

9. Emergency lowering controls should be:

☐ A. Known and functional before use

☐ B. Ignored unless a breakdown occurs

☐ C. Locked out during operation

☐ D. Used for normal descent only

10. This course alone qualifies an employee on every scissor-lift model.

☐ A. False

☐ B. True

Employee signature: _____________________________________________

INSTRUCTOR ONLY

# Answer key + coaching notes

1. A - Unstable surfaces, slopes, overload or environmental conditions  |  A scissor lift can tip because of:

2. A - Help prevent falls and must not be used as steps  |  The platform guardrails are intended to:

3. A - The lift and the work area, including overhead hazards  |  Before elevating, the operator should inspect:

4. A - Unsafe and not permitted  |  Standing on the guardrail to gain extra height is:

5. A - Manufacturer instructions and employer procedure  |  Wind limits should come from:

6. A - Clearance, slow control and often a spotter/site controls  |  Overhead crushing hazards require:

7. A - Maintaining required clearance and following site/manufacturer rules  |  Electrical hazards should be controlled by:

8. A - Be protected from falling objects and equipment movement  |  People below the lift should:

9. A - Known and functional before use  |  Emergency lowering controls should be:

10. A - False  |  This course alone qualifies an employee on every scissor-lift model.

COACHING RULE Do not treat a passing written score as proof of hands-on competency. Retrain missed critical concepts and complete workplace-specific verification where required.

TRAINING RECORD

# Attendance + completion record

Course: Scissor Lift Safety

Date: ____________   Instructor: ______________________________   Department / Location: ______________________________

| Employee Printed Name | Employee Signature | Job / Department | Date |
| --- | --- | --- | --- |

Site-specific topics / notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

PRACTICAL / WORKPLACE VERIFICATION

# Can the employee apply it?

Employee: ______________________________   Evaluator: ______________________________   Date: ____________

| Evaluation Item | Meets | Coach | N/A |
| --- | --- | --- | --- |
| Performs pre-use inspection | ☐ | ☐ | ☐ |
| Inspects floor/ground and overhead area | ☐ | ☐ | ☐ |
| Uses gate/guardrails correctly | ☐ | ☐ | ☐ |
| Maintains safe body position | ☐ | ☐ | ☐ |
| Controls travel/elevation smoothly | ☐ | ☐ | ☐ |
| Maintains overhead/electrical clearance | ☐ | ☐ | ☐ |
| Applies site barricade/spotter rules | ☐ | ☐ | ☐ |
| Demonstrates emergency stop/lowering | ☐ | ☐ | ☐ |

Comments / coaching:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Result:  ☐ Competent for evaluated task (subject to employer authorization)   ☐ More training/practice needed

Evaluator signature: ______________________________   Employee signature: ______________________________

COMPLETION CERTIFICATE

CERTIFICATE OF TRAINING

This certifies that
____________________________________________

completed the formal / awareness training portion of
Scissor Lift Safety

Date: ____________________     Instructor: ____________________
Employer / Location: __________________________________________

General/formal instruction. The employer must provide equipment-specific training, manufacturer instructions and verify safe operation for the actual lift and workplace.

REFERENCE / FINAL TRAINER CHECK

# Regulatory / source reference

Primary reference: OSHA scissor lift guidance / applicable scaffold & training requirements

https://www.osha.gov/etools/scaffolding/scissor-lifts/

## Before delivery

Confirm current OSHA and applicable state-plan requirements

Confirm manufacturer instructions and employer procedures

Insert workplace-specific hazards, controls, emergency contacts and rules

Use competent/qualified trainers or evaluators where required

Complete practical/workplace training and authorization requirements where applicable

Retain training records according to employer and regulatory requirements

FINAL REMINDER General/formal instruction. The employer must provide equipment-specific training, manufacturer instructions and verify safe operation for the actual lift and workplace.