TRAVISTILLEY.COM / TRAINING CENTER Sit-Down Counterbalance Forklift INSTRUCTOR + PRINT PACKET

| AUDIENCE | EST. TIME | REFERENCE |
| --- | --- | --- |
| Sit-down counterbalance operators | 30-40 min | 29 CFR 1910.178(l) |

IMPORTANT USE NOTE Equipment-specific formal instruction. Hands-on training and workplace evaluation on the actual truck type are required.

# What is inside this packet

Instructor quick-start and site-specific preparation checklist

Slide-by-slide trainer guide with discussion prompts

Employee handout / key takeaways

Printable 10-question knowledge check

Separate answer key and coaching notes

Attendance record

Practical / workplace evaluation checklist

Certificate wording and completion record

Primary regulatory/source reference

INSTRUCTOR QUICK-START

# Run a strong session without overcomplicating it.

| 1. PREPARE | Review the deck and packet. Insert your procedures, equipment, routes, photos, emergency contacts and site rules. |
| --- | --- |
| 2. PRESENT | Use the PowerPoint/PDF on a screen. Pause for discussion rather than reading every bullet. |
| 3. CHECK | Give the knowledge check. Review missed items and coach to the expected decision. |
| 4. VERIFY | Complete required hands-on or workplace-specific practice/evaluation with an appropriate trainer/evaluator. |
| 5. DOCUMENT | Use the attendance record, evaluation and certificate only after required training elements are complete. |

BEFORE EMPLOYEES WORK Equipment-specific formal instruction. Hands-on training and workplace evaluation on the actual truck type are required.

## Learning objectives

Recognize counterbalance truck stability and rear-swing hazards

Use correct mounting, restraint, travel and load-handling practices

Inspect the truck and respond to defects

Demonstrate safe turning, stacking, parking and dock approach

TRAINER GUIDE / MODULES 1-2

# 1. Counterbalance geometry

The counterweight balances the load across the front axle pivot.

Turning, speed, slopes, load height and off-center loads can move the combined center of gravity outside the stability area.

Rear steering creates tail swing.

DISCUSSION PROMPT Where could this fail here?

# 2. Mounting and operator restraint

Use three points of contact where applicable.

Use the operator restraint/seat belt provided by the manufacturer.

Keep body parts inside the operator compartment and never jump from a tipping sit-down truck.

DISCUSSION PROMPT What control do we use?

TRAINER GUIDE / MODULES 3-4

# 3. Travel position and steering

Carry the load low and tilted only enough for stability.

Turn smoothly and slow before corners.

Watch rear swing near racks, people, columns and doorways.

DISCUSSION PROMPT What would make you stop?

# 4. Load pickup and placement

Square to the load, level forks, enter fully and center the load.

Check load stability and capacity before lifting.

Raise only when stopped and positioned for stacking.

DISCUSSION PROMPT Where could this fail here?

TRAINER GUIDE / MODULES 5-6

# 5. Ramps, docks and trailers

Follow site rules for direction of travel on grades with loads.

Keep clear of dock edges and verify dock plates and trailer restraint.

Account for wet surfaces, debris and limited clearances.

DISCUSSION PROMPT What control do we use?

# 6. Shutdown and parking

Lower forks fully, neutralize controls, set brake and shut off power when unattended.

Park only in approved areas without blocking exits, aisles or emergency equipment.

DISCUSSION PROMPT What would make you stop?

FACILITATED DISCUSSION

# Stop-work / red-flag conditions

Seat belt/restraint is damaged or bypassed

Turning fast with a raised load

Operating close to rack uprights without accounting for tail swing

Entering a trailer without confirmed restraint/chocks and floor condition

SCENARIO A sit-down forklift begins to tip sideways during a turn. What should the seated operator do?

RECOMMENDED RESPONSE Stay in the operator compartment, hold on, brace and lean away from the direction of the tip; do not jump from a tipping sit-down counterbalance truck.

## Make it site-specific

Actual truck controls, capacity plate and attachments

Site traffic plan, pedestrian routes and speed expectations

Grade/ramp rules and dock/trailer procedures

Hands-on stacking/unstacking and workplace evaluation

EMPLOYEE HANDOUT

# Sit-Down Counterbalance Forklift

KEEP THIS PAGE

## Key decisions to remember

Counterbalance geometry: The counterweight balances the load across the front axle pivot.

Mounting and operator restraint: Use three points of contact where applicable.

Travel position and steering: Carry the load low and tilted only enough for stability.

Load pickup and placement: Square to the load, level forks, enter fully and center the load.

Ramps, docks and trailers: Follow site rules for direction of travel on grades with loads.

Shutdown and parking: Lower forks fully, neutralize controls, set brake and shut off power when unattended.

## Stop and get help when...

Seat belt/restraint is damaged or bypassed

Turning fast with a raised load

Operating close to rack uprights without accounting for tail swing

Entering a trailer without confirmed restraint/chocks and floor condition

## Our site-specific information

☐ Actual truck controls, capacity plate and attachments

☐ Site traffic plan, pedestrian routes and speed expectations

☐ Grade/ramp rules and dock/trailer procedures

☐ Hands-on stacking/unstacking and workplace evaluation

Employee notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

EMPLOYEE KNOWLEDGE CHECK

# Sit-Down Counterbalance Forklift - Knowledge Check

Employee: ____________________________________________      Date: ____________________

1. Rear-wheel steering means the operator must watch for:

☐ A. Tail swing

☐ B. Front axle lockup

☐ C. Automatic braking

☐ D. Fork spread only

2. A loaded counterbalance forklift should normally travel with the load:

☐ A. Low and stable

☐ B. Raised above eye level

☐ C. Fully tilted forward

☐ D. Resting on the floor

3. During a tip-over in a seated sit-down forklift, the operator should generally:

☐ A. Stay in the compartment and brace rather than jump

☐ B. Jump clear immediately

☐ C. Remove the seat belt

☐ D. Stand up on the seat

4. Before lifting a load, the operator should check:

☐ A. Capacity, load stability and fork engagement

☐ B. Only the destination height

☐ C. Only the clock

☐ D. Only tire pressure

5. Fast turns increase:

☐ A. Tip-over risk

☐ B. Rated capacity

☐ C. Fork strength

☐ D. Visibility

Score: ______ / 10     Passing requirement / retraining criteria: ______________________________________________

EMPLOYEE KNOWLEDGE CHECK / CONTINUED

6. When parking unattended, forks should be:

☐ A. Fully lowered

☐ B. Raised to eye level

☐ C. Left in the rack

☐ D. Tilted forward in the aisle

7. A dock plate should be entered only after:

☐ A. Its placement/capacity and trailer condition/restraint are verified

☐ B. The horn is disabled

☐ C. The load is raised

☐ D. A pedestrian waves

8. An attachment can:

☐ A. Change truck capacity and operating characteristics

☐ B. Never affect capacity

☐ C. Eliminate the need for training

☐ D. Allow overloading

9. Tail swing is most important near:

☐ A. Racks, columns, doorways and pedestrians

☐ B. Open parking lots only

☐ C. Battery chargers only

☐ D. Empty aisles only

10. This course replaces practical evaluation on the actual truck.

☐ A. False

☐ B. True

Employee signature: _____________________________________________

INSTRUCTOR ONLY

# Answer key + coaching notes

1. A - Tail swing  |  Rear-wheel steering means the operator must watch for:

2. A - Low and stable  |  A loaded counterbalance forklift should normally travel with the load:

3. A - Stay in the compartment and brace rather than jump  |  During a tip-over in a seated sit-down forklift, the operator should generally:

4. A - Capacity, load stability and fork engagement  |  Before lifting a load, the operator should check:

5. A - Tip-over risk  |  Fast turns increase:

6. A - Fully lowered  |  When parking unattended, forks should be:

7. A - Its placement/capacity and trailer condition/restraint are verified  |  A dock plate should be entered only after:

8. A - Change truck capacity and operating characteristics  |  An attachment can:

9. A - Racks, columns, doorways and pedestrians  |  Tail swing is most important near:

10. A - False  |  This course replaces practical evaluation on the actual truck.

COACHING RULE Do not treat a passing written score as proof of hands-on competency. Retrain missed critical concepts and complete workplace-specific verification where required.

TRAINING RECORD

# Attendance + completion record

Course: Sit-Down Counterbalance Forklift

Date: ____________   Instructor: ______________________________   Department / Location: ______________________________

| Employee Printed Name | Employee Signature | Job / Department | Date |
| --- | --- | --- | --- |

Site-specific topics / notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

PRACTICAL / WORKPLACE VERIFICATION

# Can the employee apply it?

Employee: ______________________________   Evaluator: ______________________________   Date: ____________

| Evaluation Item | Meets | Coach | N/A |
| --- | --- | --- | --- |
| Performs pre-use inspection | ☐ | ☐ | ☐ |
| Mounts/dismounts safely | ☐ | ☐ | ☐ |
| Uses seat belt/restraint | ☐ | ☐ | ☐ |
| Controls speed and tail swing | ☐ | ☐ | ☐ |
| Maintains correct travel/load position | ☐ | ☐ | ☐ |
| Stacks and retrieves load smoothly | ☐ | ☐ | ☐ |
| Uses safe dock/trailer procedure | ☐ | ☐ | ☐ |
| Parks correctly | ☐ | ☐ | ☐ |

Comments / coaching:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Result:  ☐ Competent for evaluated task (subject to employer authorization)   ☐ More training/practice needed

Evaluator signature: ______________________________   Employee signature: ______________________________

COMPLETION CERTIFICATE

CERTIFICATE OF TRAINING

This certifies that
____________________________________________

completed the formal / awareness training portion of
Sit-Down Counterbalance Forklift

Date: ____________________     Instructor: ____________________
Employer / Location: __________________________________________

Equipment-specific formal instruction. Hands-on training and workplace evaluation on the actual truck type are required.

REFERENCE / FINAL TRAINER CHECK

# Regulatory / source reference

Primary reference: 29 CFR 1910.178(l)

https://www.osha.gov/etools/powered-industrial-trucks/training

## Before delivery

Confirm current OSHA and applicable state-plan requirements

Confirm manufacturer instructions and employer procedures

Insert workplace-specific hazards, controls, emergency contacts and rules

Use competent/qualified trainers or evaluators where required

Complete practical/workplace training and authorization requirements where applicable

Retain training records according to employer and regulatory requirements

FINAL REMINDER Equipment-specific formal instruction. Hands-on training and workplace evaluation on the actual truck type are required.