TRAVISTILLEY.COM / TRAINING CENTER Stand-Up Reach Truck INSTRUCTOR + PRINT PACKET

| AUDIENCE | EST. TIME | REFERENCE |
| --- | --- | --- |
| Reach-truck operators | 30-40 min | 29 CFR 1910.178(l) |

IMPORTANT USE NOTE Equipment-specific formal instruction. Practical training and workplace evaluation on the reach truck are required.

# What is inside this packet

Instructor quick-start and site-specific preparation checklist

Slide-by-slide trainer guide with discussion prompts

Employee handout / key takeaways

Printable 10-question knowledge check

Separate answer key and coaching notes

Attendance record

Practical / workplace evaluation checklist

Certificate wording and completion record

Primary regulatory/source reference

INSTRUCTOR QUICK-START

# Run a strong session without overcomplicating it.

| 1. PREPARE | Review the deck and packet. Insert your procedures, equipment, routes, photos, emergency contacts and site rules. |
| --- | --- |
| 2. PRESENT | Use the PowerPoint/PDF on a screen. Pause for discussion rather than reading every bullet. |
| 3. CHECK | Give the knowledge check. Review missed items and coach to the expected decision. |
| 4. VERIFY | Complete required hands-on or workplace-specific practice/evaluation with an appropriate trainer/evaluator. |
| 5. DOCUMENT | Use the attendance record, evaluation and certificate only after required training elements are complete. |

BEFORE EMPLOYEES WORK Equipment-specific formal instruction. Practical training and workplace evaluation on the reach truck are required.

## Learning objectives

Recognize narrow-aisle and stand-up operator hazards

Control travel, turning and load placement at height

Use correct stance, body positioning and visibility practices

Inspect and operate reach mechanisms safely

TRAINER GUIDE / MODULES 1-2

# 1. Reach-truck differences

Narrow-aisle design changes visibility, steering feel and clearance needs.

Stand-up operator position creates different egress and impact hazards.

Reach mechanisms and high stacking require precise load control.

DISCUSSION PROMPT Where could this fail here?

# 2. Operator position

Stay within the protected operator compartment.

Keep feet and body parts inside the truck profile.

Use the manufacturer-prescribed stance and controls.

DISCUSSION PROMPT What control do we use?

TRAINER GUIDE / MODULES 3-4

# 3. Narrow-aisle travel

Control speed before entering turns and cross aisles.

Account for rack uprights, end-of-aisle traffic and pedestrians.

Travel in the direction that provides best safe visibility based on truck design and site rules.

DISCUSSION PROMPT What would make you stop?

# 4. High stacking

Square to the rack before raising.

Verify rack capacity/condition and pallet integrity.

Raise while stopped; use reach/tilt smoothly and watch overhead clearance.

DISCUSSION PROMPT Where could this fail here?

TRAINER GUIDE / MODULES 5-6

# 5. Stability and loads

Know capacity at lift height and load center.

Avoid sudden steering, braking or mast movement with elevated loads.

Do not travel with the load unnecessarily elevated.

DISCUSSION PROMPT What control do we use?

# 6. Inspection and defects

Inspect forks, mast, reach mechanism, chains, wheels, controls, battery, horn and safety systems.

Remove the truck from service for conditions that affect safe operation.

DISCUSSION PROMPT What would make you stop?

FACILITATED DISCUSSION

# Stop-work / red-flag conditions

Foot or leg extends outside the operator compartment

Traveling with elevated load in a narrow aisle

Racking is visibly damaged or pallet integrity is questionable

Operator loses clear view of pedestrians at aisle ends

SCENARIO A reach-truck operator has a pallet raised near the top rack and needs to reposition the truck sideways. What is the safe approach?

RECOMMENDED RESPONSE Lower the load to a stable travel position before repositioning unless the manufacturer/site procedure specifically allows a controlled adjustment at height; avoid traveling with a high load.

## Make it site-specific

Truck-specific controls and emergency features

Aisle widths, intersections, speed limits and pedestrian controls

Rack inspection/damage reporting and pallet standards

Hands-on high stacking and workplace evaluation

EMPLOYEE HANDOUT

# Stand-Up Reach Truck

KEEP THIS PAGE

## Key decisions to remember

Reach-truck differences: Narrow-aisle design changes visibility, steering feel and clearance needs.

Operator position: Stay within the protected operator compartment.

Narrow-aisle travel: Control speed before entering turns and cross aisles.

High stacking: Square to the rack before raising.

Stability and loads: Know capacity at lift height and load center.

Inspection and defects: Inspect forks, mast, reach mechanism, chains, wheels, controls, battery, horn and safety systems.

## Stop and get help when...

Foot or leg extends outside the operator compartment

Traveling with elevated load in a narrow aisle

Racking is visibly damaged or pallet integrity is questionable

Operator loses clear view of pedestrians at aisle ends

## Our site-specific information

☐ Truck-specific controls and emergency features

☐ Aisle widths, intersections, speed limits and pedestrian controls

☐ Rack inspection/damage reporting and pallet standards

☐ Hands-on high stacking and workplace evaluation

Employee notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

EMPLOYEE KNOWLEDGE CHECK

# Stand-Up Reach Truck - Knowledge Check

Employee: ____________________________________________      Date: ____________________

1. A stand-up reach truck operator should keep body parts:

☐ A. Inside the protected operator compartment

☐ B. Outside the truck for better visibility

☐ C. Against rack uprights

☐ D. On the load backrest

2. High stacking should be done with the truck:

☐ A. Stopped and squared to the rack

☐ B. Moving through the aisle

☐ C. Turning sharply

☐ D. On a slope

3. Traveling with a high elevated load:

☐ A. Increases instability and should be avoided

☐ B. Improves stability

☐ C. Is required in narrow aisles

☐ D. Eliminates blind spots

4. Before placing a pallet high in racking, verify:

☐ A. Pallet/rack condition and truck capacity at height

☐ B. Only the pallet label

☐ C. Only battery charge

☐ D. Only the aisle number

5. A narrow aisle requires extra attention to:

☐ A. Rack clearance, pedestrians and end-of-aisle traffic

☐ B. Open-road speed

☐ C. Trailer tire pressure

☐ D. Outdoor weather only

Score: ______ / 10     Passing requirement / retraining criteria: ______________________________________________

EMPLOYEE KNOWLEDGE CHECK / CONTINUED

6. Reach mechanism defects should result in:

☐ A. Truck removal from service until safe

☐ B. Slower operation only

☐ C. Higher travel height

☐ D. Use by experienced operators only

7. Sudden steering with an elevated load:

☐ A. Can destabilize the truck

☐ B. Improves steering response

☐ C. Increases capacity

☐ D. Is recommended

8. Operator stance should follow:

☐ A. Manufacturer design and employer training

☐ B. Personal preference only

☐ C. The same stance as a sit-down truck

☐ D. No specific guidance

9. Rack damage should be:

☐ A. Reported and controlled per site procedure

☐ B. Ignored if the pallet fits

☐ C. Covered with paint

☐ D. Used only for light loads

10. This course alone authorizes reach-truck operation.

☐ A. False

☐ B. True

Employee signature: _____________________________________________

INSTRUCTOR ONLY

# Answer key + coaching notes

1. A - Inside the protected operator compartment  |  A stand-up reach truck operator should keep body parts:

2. A - Stopped and squared to the rack  |  High stacking should be done with the truck:

3. A - Increases instability and should be avoided  |  Traveling with a high elevated load:

4. A - Pallet/rack condition and truck capacity at height  |  Before placing a pallet high in racking, verify:

5. A - Rack clearance, pedestrians and end-of-aisle traffic  |  A narrow aisle requires extra attention to:

6. A - Truck removal from service until safe  |  Reach mechanism defects should result in:

7. A - Can destabilize the truck  |  Sudden steering with an elevated load:

8. A - Manufacturer design and employer training  |  Operator stance should follow:

9. A - Reported and controlled per site procedure  |  Rack damage should be:

10. A - False  |  This course alone authorizes reach-truck operation.

COACHING RULE Do not treat a passing written score as proof of hands-on competency. Retrain missed critical concepts and complete workplace-specific verification where required.

TRAINING RECORD

# Attendance + completion record

Course: Stand-Up Reach Truck

Date: ____________   Instructor: ______________________________   Department / Location: ______________________________

| Employee Printed Name | Employee Signature | Job / Department | Date |
| --- | --- | --- | --- |

Site-specific topics / notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

PRACTICAL / WORKPLACE VERIFICATION

# Can the employee apply it?

Employee: ______________________________   Evaluator: ______________________________   Date: ____________

| Evaluation Item | Meets | Coach | N/A |
| --- | --- | --- | --- |
| Inspects reach mechanism and safety systems | ☐ | ☐ | ☐ |
| Maintains body position inside compartment | ☐ | ☐ | ☐ |
| Controls speed in narrow aisles | ☐ | ☐ | ☐ |
| Squares to racks before lifting | ☐ | ☐ | ☐ |
| Uses reach/tilt smoothly | ☐ | ☐ | ☐ |
| Places/retrieves load without rack contact | ☐ | ☐ | ☐ |
| Maintains safe travel height | ☐ | ☐ | ☐ |
| Parks/charges safely | ☐ | ☐ | ☐ |

Comments / coaching:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Result:  ☐ Competent for evaluated task (subject to employer authorization)   ☐ More training/practice needed

Evaluator signature: ______________________________   Employee signature: ______________________________

COMPLETION CERTIFICATE

CERTIFICATE OF TRAINING

This certifies that
____________________________________________

completed the formal / awareness training portion of
Stand-Up Reach Truck

Date: ____________________     Instructor: ____________________
Employer / Location: __________________________________________

Equipment-specific formal instruction. Practical training and workplace evaluation on the reach truck are required.

REFERENCE / FINAL TRAINER CHECK

# Regulatory / source reference

Primary reference: 29 CFR 1910.178(l)

https://www.osha.gov/etools/powered-industrial-trucks/workplace/narrow-aisles

## Before delivery

Confirm current OSHA and applicable state-plan requirements

Confirm manufacturer instructions and employer procedures

Insert workplace-specific hazards, controls, emergency contacts and rules

Use competent/qualified trainers or evaluators where required

Complete practical/workplace training and authorization requirements where applicable

Retain training records according to employer and regulatory requirements

FINAL REMINDER Equipment-specific formal instruction. Practical training and workplace evaluation on the reach truck are required.